| Suarez Company |  | Revision nr. 2 Dated 16/01/2025 Printed on 16/01/2025 1/31 Replaced revision:1 (Dated: 04/09/2023) |  |
| --- | --- | --- | --- |
| BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO |  |  |  |
| Safety Data Sheet According to Annex II to REACH - Regulation (EU) 2020/878 |  |  |  |
| SECTION 1. Identification of the substance/mixture and of the company/undertaking |  |  |  |
| - Product identifier Code: BLK0035-2 Product name SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO UFI : 19WA-P3AW-QX01-0EQK - Relevant identified uses of the substance or mixture and uses advised against |  |  |  |
| Identified Uses | Industrial Professional | Consumer |  |
| Air freshener | - - |  |  |
| Uses Advised Against |  |  |  |
| Do not use for uses other than those indicated |  |  |  |
| 1.3. Details of the supplier of the safety data sheet Name Full address District and Country | Suarez Company Via Pergolesi , 1 ang. Via Monteverdi 20811 Cesano Maderno Italia |  |  |
|  | Tel. +39 0362 659766 |  |  |
|  | www.suarezcompany.it |  |  |
| e-mail address of the competent person |  |  |  |
| responsible for the Safety Data Sheet | info@suarezcompany.it |  |  |
| 1.4. Emergency telephone number For urgent inquiries refer to | Suarez Company |  |  |
|  | +39 0362 659766 |  |  |
|  | (8:30 - 12:30 13:30-17:30) |  |  |
| SECTION 2. Hazards identification |  |  |  |
| 2.1. Classification of the substance or mixture The product is classified as hazardous pursuant to the provisions set forth in (EC) Regulation 1272/2008 (CLP) (and subsequent amendments and supplements). The product thus requires a safety datasheet that complies with the provisions of (EU) Regulation 2020/878. Any additional information concerning the risks for health and/or the environment are given in sections 11 and 12 of this sheet. Hazard classification and indication: Flammable liquid, category 2 H225 Highly flammable liquid and vapour. Eye irritation, category 2 H319 Causes serious eye irritation. Skin sensitization, category 1A H317 May cause an allergic skin reaction. Hazardous to the aquatic environment, chronic toxicity, H412 Harmful to aquatic life with long lasting effects. category 3 |  |  |  |

# BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO

Printed on 16/01/2025

2/31

Replaced revision:1 (Dated: 04/09/2023)

2.2. Label elements

Hazard labelling pursuant to EC Regulation 1272/2008 (CLP) and subsequent amendments and supplements. Hazard pictograms:

| Signal words: | Danger |
| --- | --- |

| Hazard statements: |  |
| --- | --- |
| H225 | Highly flammable liquid and vapour. |

| H319 | Causes serious eye irritation. |
| --- | --- |

| H317 | May cause an allergic skin reaction. |
| --- | --- |

| H412 | Harmful to aquatic life with long lasting effects. |
| --- | --- |

| Precautionary statements: |  |
| --- | --- |
| P101 | If medical advice is needed, have product container or label at hand. |

| P102 | Keep out of reach of children. |
| --- | --- |

| P210 | Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. |
| --- | --- |

| P305+P351+P338 | IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. |
| --- | --- |

| P337+P313 | If eye irritation persists: Get medical advice / attention. |
| --- | --- |

| P501 | Dispose of contents / container in accordance with current regulations. |
| --- | --- |

| Contains: | Isoeugenol |
| --- | --- |
|  | 3,7-DIMETHYLOCT-6-EN-1-OL |
|  | 3,7-DIMETHYLNONA-1,6-DIEN-3-OL |
|  | 2H-CHROMEN-2-ONE |
|  | geraniol |
|  | LINALYL ACETATE |

2.3. Other hazards

# BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO

Printed on 16/01/2025

3/31

Replaced revision:1 (Dated: 04/09/2023)

On the basis of available data, the product does not contain any PBT or vPvB in percentage ≥ than 0,1%. The product does not contain substances with endocrine disrupting properties in concentration ≥ 0.1%.

## SECTION 3. Composition/information on ingredients

  - Substances

Information not relevant

  - Mixtures

Contains:

| Identification | x = Conc. % | Classification (EC) 1272/2008 (CLP) |
| --- | --- | --- |
| ETHANOL |  |  |
| INDEX 603-002-00-5 | 74 ≤ x < 78 | Flam. Liq. 2 H225, Eye Irrit. 2 H319 |
| EC 200-578-6 |  | Eye Irrit. 2 H319: ≥ 50% |
| CAS 64-17-5 |  |  |
| REACH Reg. 01-2119457610-43 |  |  |
| DIPROPYLENE GLYCOL MONOMETHYL ETHER |  |  |
| INDEX - | 7 ≤ x < 8 | Substance with a community workplace exposure limit. |
| EC 252-104-2 |  |  |
| CAS 34590-94-8 |  |  |
| REACH Reg. 01-2119450011-60 |  |  |
| ALPHA-ISOMETHYL IONONE |  |  |
| INDEX - | 1 ≤ x < 1,5 | Aquatic Chronic 2 H411 |
| EC 204-846-3 |  |  |
| CAS 127-51-5 |  |  |
| REACH Reg. 01-2120138569-45 |  |  |
| galaxolide |  |  |
| INDEX 603-212-00-7 | 0,6 ≤ x < 0,7 | Aquatic Acute 1 H400 M=1, Aquatic Chronic 1 H410 M=1 |
| EC 214-946-9 |  |  |
| CAS 1222-05-5 |  |  |
| REACH Reg. 01-2119488227-29 |  |  |
| LINALYL ACETATE |  |  |
| INDEX - | 0,4 ≤ x < 0,45 | Eye Irrit. 2 H319, Skin Irrit. 2 H315, Skin Sens. 1B H317 |
| EC 204-116-4 |  |  |
| CAS 115-95-7 |  |  |
| REACH Reg. 01-2119454789-19 |  |  |
| geraniol |  |  |
| INDEX 603-241-00-5 | 0,3 ≤ x < 0,35 | Eye Dam. 1 H318, Skin Irrit. 2 H315, Skin Sens. 1 H317 |
| EC 203-377-1 |  |  |
| CAS 106-24-1 |  |  |
| REACH Reg. 01-2119552430-49 |  |  |

# BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO

Printed on 16/01/2025

4/31

Replaced revision:1 (Dated: 04/09/2023)

| 2H-CHROMEN-2-ONE |  |  |
| --- | --- | --- |
| INDEX - | 0,3 ≤ x < 0,35 | Acute Tox. 4 H302, Skin Sens. 1 H317, Aquatic Chronic 3 H412 |
| EC 202-086-7 |  | ATE Oral: 500 mg/kg |
| CAS 91-64-5 |  |  |
| REACH Reg. 01-2119943756-26 |  |  |
| 3,7-DIMETHYLNONA-1,6-DIEN-3-OL |  |  |
| INDEX - | 0,25 ≤ x < 0,3 | Eye Irrit. 2 H319, Skin Irrit. 2 H315, Skin Sens. 1B H317 |
| EC 233-732-6 |  |  |
| CAS 10339-55-6 |  |  |
| REACH Reg. 01-2119969272-32 |  |  |
| 3,7-DIMETHYLOCT-6-EN-1-OL |  |  |
| INDEX - | 0,2 ≤ x < 0,25 | Eye Irrit. 2 H319, Skin Irrit. 2 H315, Skin Sens. 1B H317 |
| EC 203-375-0 |  |  |
| CAS 106-22-9 |  |  |
| REACH Reg. 01-2119453995-23 |  |  |
| Isoeugenol |  |  |
| INDEX 604-094-00-X | 0,1 ≤ x < 0,15 | Skin Sens. 1A H317 |
| EC 202-590-7 |  | Skin Sens. 1A H317: ≥ 0,01% |
| CAS 97-54-1 |  |  |
| METHANOL |  |  |
| INDEX 603-001-00-X | 0 < x < 0,05 | Flam. Liq. 2 H225, Acute Tox. 3 H301, Acute Tox. 3 H311, Acute Tox. 3 H331, STOT SE 1 H370 |
| EC 200-659-6 |  | STOT SE 2 H371: ≥ 3% - < 10% |
| CAS 67-56-1 |  | ATE Oral: 100 mg/kg, ATE Dermal: 300 mg/kg, ATE Inhalation vapours: 3 mg/l |
| REACH Reg. 01-2119433307-44 |  |  |
| toluene |  |  |
| INDEX 601-021-00-3 | 0 < x < 0,05 | Flam. Liq. 2 H225, Repr. 2 H361d, Asp. Tox. 1 H304, STOT RE 2 H373, Skin Irrit. 2 H315, STOT SE 3 H336 |
| EC 203-625-9 |  |  |
| CAS 108-88-3 |  |  |
| REACH Reg. 01-2119471310-51- |  |  |
| ACETONE |  |  |
| INDEX 606-001-00-8 | 0 < x < 0,05 | Flam. Liq. 2 H225, Eye Irrit. 2 H319, STOT SE 3 H336, EUH066 |
| EC 200-662-2 |  |  |
| CAS 67-64-1 |  |  |
| REACH Reg. 01-2119471330-49 |  |  |
| XYLENE |  |  |
| INDEX 601-022-00-9 | 0 < x < 0,05 | Flam. Liq. 3 H226, Acute Tox. 4 H312, Acute Tox. 4 H332, Skin Irrit. 2 H315, Classification note according to Annex VI to the CLP Regulation: C |
| EC 202-422-2 |  | ATE Dermal: 1100 mg/kg, ATE Inhalation vapours: 11 mg/l |
| CAS 95-47-6 |  |  |
| REACH Reg. 01-2119485822-30 |  |  |

The full wording of hazard (H) phrases is given in section 16 of the sheet.

| Suarez Company | Revision nr. 2 Dated 16/01/2025 Printed on 16/01/2025 5/31 Replaced revision:1 (Dated: 04/09/2023) |
| --- | --- |
| BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO |  |
| SECTION 4. First aid measures - Description of first aid measures In case of doubt or in the presence of symptoms contact a doctor and show him this document. In case of more severe symptoms, ask for immediate medical aid. EYES: Remove, if present, contact lenses if the situation allows you to do so easily. Wash immediately with plenty of water for at least 15 minutes, opening the eyelids fully. Get medical advice/attention. SKIN: Take off immediately all contaminated clothing. Wash immediately and thoroughly with running water (and soap if possible). Get medical advice/attention. Avoid further contact with contaminated clothing. INGESTION: Do not induce vomiting unless explicitly authorised by a doctor. Do not give anything by mouth to an unconscious person. Get medical advice/attention. INHALATION: Remove victim to fresh air, away from the accident scene. Get medical advice/attention. Rescuer protection It is good practice for rescuers lending support to a person who has been exposed to a chemical substance or to a mixture to wear personal protective equipment. The nature of such protection depends on the hazard level of the substance or mixture, on the type of exposure and on the extent of the contamination. In the absence of other more specific indications, use of disposable gloves in the event of possible contact with body fluids is recommended. For the type of PPE suitable for the characteristics of the substance or mixture, see section 8. - Most important symptoms and effects, both acute and delayed Specific information on symptoms and effects caused by the product are unknown. DELAYED EFFECTS: Based on the information currently available, there are no known cases of delayed effects following exposure to this product. - Indication of any immediate medical attention and special treatment needed If symptoms occur, whether acute or delayed, consult a doctor. Means to have available in the workplace for specific and immediate treatment Running water for skin and eye wash. SECTION 5. Firefighting measures - Extinguishing media SUITABLE EXTINGUISHING EQUIPMENT Extinguishing substances are: carbon dioxide, foam, chemical powder. For product loss or leakage that has not caught fire, water spray can be used to disperse flammable vapours and protect those trying to stem the leak. UNSUITABLE EXTINGUISHING EQUIPMENT Do not use jets of water. Water is not effective for putting out fires but can be used to cool containers exposed to flames to prevent explosions. - Special hazards arising from the substance or mixture HAZARDS CAUSED BY EXPOSURE IN THE EVENT OF FIRE Excess pressure may form in containers exposed to fire at a risk of explosion. Do not breathe combustion products. - Advice for firefighters GENERAL INFORMATION Use jets of water to cool the containers to prevent product decomposition and the development of substances potentially hazardous for health. Always |  |

| Suarez Company | Revision nr. 2 Dated 16/01/2025 Printed on 16/01/2025 6/31 Replaced revision:1 (Dated: 04/09/2023) |
| --- | --- |
| BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO |  |
| wear full fire prevention gear. Collect extinguishing water to prevent it from draining into the sewer system. Dispose of contaminated water used for extinction and the remains of the fire according to applicable regulations. SPECIAL PROTECTIVE EQUIPMENT FOR FIRE-FIGHTERS Normal fire fighting clothing i.e. fire kit (BS EN 469), gloves (BS EN 659) and boots (HO specification A29 and A30) in combination with self-contained open circuit positive pressure compressed air breathing apparatus (BS EN 137). SECTION 6. Accidental release measures - Personal precautions, protective equipment and emergency procedures Block the leakage if there is no hazard. Wear suitable protective equipment (including personal protective equipment referred to under Section 8 of the safety data sheet) to prevent any contamination of skin, eyes and personal clothing. These indications apply for both processing staff and those involved in emergency procedures. Send away individuals who are not suitably equipped. Use explosion-proof equipment. Eliminate all sources of ignition (cigarettes, flames, sparks, etc.) from the leakage site. - Environmental precautions The product must not penetrate into the sewer system or come into contact with surface water or ground water. - Methods and material for containment and cleaning up Collect the leaked product into a suitable container. Evaluate the compatibility of the container to be used, by checking section 10. Absorb the remainder with inert absorbent material. Make sure the leakage site is well aired. Contaminated material should be disposed of in compliance with the provisions set forth in point 13. - Reference to other sections Any information on personal protection and disposal is given in sections 8 and 13. SECTION 7. Handling and storage - Precautions for safe handling Keep away from heat, sparks and naked flames; do not smoke or use matches or lighters. Without adequate ventilation, vapours may accumulate at ground level and, if ignited, catch fire even at a distance, with the danger of backfire. Avoid bunching of electrostatic charges. When performing transfer operations involving large containers, connect to an earthing system and wear antistatic footwear. Vigorous stirring and flow through the tubes and equipment may cause the formation and accumulation of electrostatic charges. In order to avoid the risk of fires and explosions, never use compressed air when handling. Open containers with caution as they may be pressurised. Do not eat, drink or smoke during use. Avoid leakage of the product into the environment. - Conditions for safe storage, including any incompatibilities Store only in the original container. Store the containers sealed, in a well ventilated place, away from direct sunlight. Store in a cool and well ventilated place, keep far away from sources of heat, naked flames and sparks and other sources of ignition. Keep containers away from any incompatible materials, see section 10 for details. Storage class TRGS 510 (Germany): 3 - Specific end use(s) |  |

# Suarez Company

BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO

Revision nr. 2 Dated 16/01/2025

Printed on 16/01/2025

7/31

Replaced revision:1 (Dated: 04/09/2023)

Information not available

## SECTION 8. Exposure controls/personal protection

8.1. Control parameters

Regulatory references:

| BGR | България | НАРЕДБА № 13 ОТ 30 ДЕКЕМВРИ 2003 Г. ЗА ЗАЩИТА НА РАБОТЕЩИТЕ ОТ РИСКОВЕ, СВЪРЗАНИ С ЕКСПОЗИЦИЯ НА ХИМИЧНИ АГЕНТИ ПРИ РАБОТА (изм. ДВ. бр.5 от 17 Януари 2020г.) |
| --- | --- | --- |
| CZE | Česká Republika | NAŘÍZENÍ VLÁDY ze dne 10. května 2021, kterým se mění nařízení vlády č. 361/2007 Sb., kterým se stanoví podmínky ochrany zdraví při práci |
| DEU | Deutschland | Forschungsgemeinschaft MAK- und BAT-Werte-Liste 2022 Ständige Senatskommission zur Prüfung gesundheitsschädlicher Arbeitsstoffe Mitteilung 58 |
| DNK | Danmark | Bekendtgørelse om grænseværdier for stoffer og materialer - BEK nr 1458 af 13/12/2019 |
| ESP | España | Límites de exposición profesional para agentes químicos en España 2023 |
| FRA | France | Valeurs limites d'exposition professionnelle aux agents chimiques en FranceDécret n° 2021-1849 du 28 décembre 2021 |
| FIN | Suomi | HTP-VÄRDEN 2020. Koncentrationer som befunnits skadliga. SOCIAL - OCH HÄLSOVÅRDSMINISTERIETS PUBLIKATIONER 2020:25 |
| GRC | Ελλάδα | Π.Δ. 26/2020 (ΦΕΚ 50/Α` 6.3.2020) Εναρμόνιση της ελληνικής νομοθεσίας προς τις διατάξεις των οδηγιών 2017/2398/EΕ, 2019/130/ΕΕ και 2019/983/ΕΕ «για την τροποποίηση της οδηγίας 2004/37/ΕΚ ``σχετικά με την προστασία των εργαζομένων από τους κινδύνους που συνδέονται με την έκθεση σε καρκινογόνους ή μεταλλαξιγόνους παράγοντες κατά την εργασία``» |
| HUN | Magyarország | Az innovációért és technológiáért felelős miniszter 5/2020. (II. 6.) ITM rendelete a kémiai kóroki tényezők hatásának kitett munkavállalók egészségének és biztonságának védelméről |
| HRV | Hrvatska | Pravilnik o izmjenama i dopunama Pravilnika o zaštiti radnika od izloženosti opasnimkemikalijama na radu, graničnim vrijednostima izloženosti i biološkim graničnim vrijednostima (NN 1/2021) |
| ITA | Italia | Decreto Legislativo 9 Aprile 2008, n.81 |
| NOR | Norge | Forskrift om endring i forskrift om tiltaksverdier og grenseverdier for fysiske og kjemiske faktorer i arbeidsmiljøet samt smitterisikogrupper for biologiske faktorer (forskrift om tiltaks- og grenseverdier), 21. august 2018 nr. 1255 |
| NLD | Nederland | Arbeidsomstandighedenregeling. Lijst van wettelijke grenswaarden op grond van de artikelen 4.3, eerste lid, en 4.16, eerste lid, van het Arbeidsomstandighedenbesluit |
| PRT | Portugal | Decreto-Lei n.º 1/2021 de 6 de janeiro, valores-limite de exposição profissional indicativos para os agentes químicos. Decreto-Lei n.º 35/2020 de 13 de julho, proteção dos trabalhadores contra os riscos ligados à exposição durante o trabalho a agentes cancerígenos ou mutagénicos |
| POL | Polska | Rozporządzenie ministra rozwoju, pracy i technologii z dnia 18 lutego 2021 r. Zmieniające rozporządzenie w sprawie najwyższych dopuszczalnych stężeń i natężeń czynników szkodliwych dla zdrowia w środowisku pracy |
| ROU | România | Hotărârea nr. 53/2021 pentru modificarea hotărârii guvernului nr. 1.218/2006, precum și pentru modificarea și completarea hotărârii guvernului nr. 1.093/2006 |
| SWE | Sverige | Hygieniska gränsvärden, Arbetsmiljöverkets föreskrifter och allmänna råd om hygieniska gränsvärden (AFS 2018:1) |
| SVK | Slovensko | NARIADENIE VLÁDY Slovenskej republiky z 12. augusta 2020, ktorým sa mení a dopĺňa nariadenie vlády Slovenskej republiky č. 356/2006 Z. z. o ochrane zdravia zamestnancov pred rizikami súvisiacimi s expozíciou karcinogénnym a mutagénnym faktorom pri práci v znení neskorších predpisov |
| SVN | Slovenija | Pravilnik o varovanju delavcev pred tveganji zaradi izpostavljenosti kemičnim snovem pri delu (Uradni list RS, št. 100/01, 39/05, 53/07, 102/10, 43/11 – ZVZD-1, 38/15, 78/18 in 78/19) |
| GBR | United Kingdom | EH40/2005 Workplace exposure limits (Fourth Edition 2020) |
| EU | OEL EU | Directive (EU) 2022/431; Directive (EU) 2019/1831; Directive (EU) 2019/130; Directive (EU) 2019/983; Directive (EU) 2017/2398; Directive (EU) 2017/164; Directive 2009/161/EU; Directive 2006/15/EC; Directive 2004/37/EC; Directive 2000/39/EC; Directive 98/24/EC; Directive 91/322/EEC. |
|  | TLV-ACGIH | ACGIH 2023 |

ETHANOL

Threshold Limit Value

Type	Country	TWA/8h	STEL/15min	Remarks / Observations

mg/m3	ppm	mg/m3	ppm

TLV	BGR	1000

TLV	CZE	1000	3000

AGW	DEU	960	500	1920	1000

| Suarez Company |  |  |  |  |  |  |  | Revision nr. 2 Dated 16/01/2025 Printed on 16/01/2025 8/31 Replaced revision:1 (Dated: 04/09/2023) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO |  |  |  |  |  |  |  |  |  |  |
| MAK | DEU | 960 |  | 500 | 1920 | 1000 |  |  |  |  |
| TLV | DNK | 1900 |  | 1000 |  |  |  |  |  |  |
| VLA | ESP |  |  |  | 1910 | 1000 |  |  |  |  |
| VLEP | FRA | 1900 |  | 1000 | 9500 | 5000 |  |  |  |  |
| HTP | FIN | 1900 |  | 1000 | 2500 | 1300 |  |  |  |  |
| TLV | GRC | 1900 |  | 1000 |  |  |  |  |  |  |
| AK | HUN | 1900 |  |  | 7600 |  |  |  |  |  |
| GVI/KGVI | HRV | 1900 |  | 1000 |  |  |  |  |  |  |
| TLV | NOR | 950 |  | 500 |  |  |  |  |  |  |
| TGG | NLD | 260 |  |  | 1900 |  |  |  |  |  |
| NDS/NDSCh | POL | 1900 |  |  |  |  |  |  |  |  |
| NGV/KGV | SWE | 1000 |  | 500 | 1900 | 1000 |  |  |  |  |
| NPEL | SVK | 960 |  | 500 | 1920 |  |  |  |  |  |
| WEL | GBR | 1920 |  | 1000 |  |  |  |  |  |  |
| TLV-ACGIH |  |  |  |  | 1884 | 1000 |  |  |  |  |
| Predicted no-effect concentration - PNEC |  |  |  |  |  |  |  |  |  |  |
| Normal value in fresh water |  |  |  |  | 0,96 | mg/l |  |  |  |  |
| Normal value in marine water |  |  |  |  | 0,79 | mg/l |  |  |  |  |
| Normal value for fresh water sediment |  |  |  |  | 3,6 | mg/kg |  |  |  |  |
| Normal value for marine water sediment |  |  |  |  | 2,9 | mg/kg |  |  |  |  |
| Normal value for water, intermittent release |  |  |  |  | 2,75 | mg/l |  |  |  |  |
| Normal value of STP microorganisms |  |  |  |  | 580 | mg/l |  |  |  |  |
| Normal value for the food chain (secondary poisoning) |  |  |  |  | 0,38 | mg/kg |  |  |  |  |
| Normal value for the terrestrial compartment |  |  |  |  | 0,63 | mg/kg |  |  |  |  |
| Health - Derived no-effect level - DNEL / DMEL |  |  |  |  |  |  |  |  |  |  |
|  | Effects on consumers |  |  |  |  | Effects on workers |  |  |  |  |
| Route of exposure | Acute local |  | Acute systemic | Chronic local | Chronic systemic | Acute local | Acute systemic | Chronic local | Chronic systemic |  |
| Oral |  |  |  |  | 87 mg/kg bw/d |  |  |  |  |  |
| Inhalation |  |  |  |  | 114 mg/m3 |  |  |  | 950 mg/m3 |  |
| Skin |  |  |  |  | 206 mg/kg bw/d |  |  |  | 343 mg/kg bw/d |  |
| DIPROPYLENE GLYCOL MONOMETHYL ETHER |  |  |  |  |  |  |  |  |  |  |
| Threshold Limit Value |  |  |  |  |  |  |  |  |  |  |
| Type | Country | TWA/8h |  |  | STEL/15min |  | Remarks / Observations |  |  |  |
|  |  | mg/m3 |  | ppm | mg/m3 | ppm |  |  |  |  |
| TLV | BGR | 308 |  |  |  |  | SKIN |  |  |  |
| TLV | CZE | 270 |  |  | 550 |  | SKIN |  |  |  |
| AGW | DEU | 310 |  | 50 | 310 | 50 |  |  |  |  |
| MAK | DEU | 310 |  | 50 | 310 | 50 |  |  |  |  |
| TLV | DNK | 300 |  | 50 |  |  |  |  |  |  |
| VLA | ESP | 308 |  | 50 |  |  | SKIN |  |  |  |
| VLEP | FRA | 308 |  | 50 |  |  | SKIN |  |  |  |

| Suarez Company |  |  |  |  |  |  |  | Revision nr. 2 Dated 16/01/2025 Printed on 16/01/2025 9/31 Replaced revision:1 (Dated: 04/09/2023) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO |  |  |  |  |  |  |  |  |  |  |
| HTP | FIN | 310 |  | 50 |  |  |  |  |  |  |
| TLV | GRC | 600 |  | 100 | 900 | 150 |  |  |  |  |
| AK | HUN | 308 |  |  | 308 |  |  |  |  |  |
| VLEP | ITA | 308 |  | 50 |  |  | SKIN |  |  |  |
| TLV | NOR | 300 |  | 50 |  |  | SKIN |  |  |  |
| VLE | PRT | 308 |  | 50 |  |  | SKIN |  |  |  |
| NDS/NDSCh | POL | 240 |  |  | 480 |  |  |  |  |  |
| NGV/KGV | SWE | 300 |  | 50 | 450 | 75 | SKIN |  |  |  |
| NPEL | SVK | 308 |  | 50 |  |  | SKIN |  |  |  |
| MV | SVN | 308 |  | 50 |  |  | SKIN |  |  |  |
| WEL | GBR | 308 |  | 50 |  |  | SKIN |  |  |  |
| OEL | EU | 308 |  | 50 |  |  | SKIN |  |  |  |
| TLV-ACGIH |  | 606 |  | 100 | 909 | 150 | SKIN |  |  |  |
| Predicted no-effect concentration - PNEC |  |  |  |  |  |  |  |  |  |  |
| Normal value in fresh water |  |  |  |  | 19 | mg/l |  |  |  |  |
| Normal value in marine water |  |  |  |  | 1,9 | mg/l |  |  |  |  |
| Normal value for fresh water sediment |  |  |  |  | 70,2 | mg/kg |  |  |  |  |
| Normal value for marine water sediment |  |  |  |  | 7,02 | mg/kg |  |  |  |  |
| Normal value for water, intermittent release |  |  |  |  | 190 | mg/l |  |  |  |  |
| Normal value of STP microorganisms |  |  |  |  | 4168 | mg/l |  |  |  |  |
| Normal value for the terrestrial compartment |  |  |  |  | 2,74 | mg/kg |  |  |  |  |
| Health - Derived no-effect level - DNEL / DMEL |  |  |  |  |  |  |  |  |  |  |
|  | Effects on consumers |  |  |  |  | Effects on workers |  |  |  |  |
| Route of exposure | Acute local |  | Acute systemic | Chronic local | Chronic systemic | Acute local | Acute systemic | Chronic local | Chronic systemic |  |
| Oral |  |  |  |  | 36 mg/kg bw/d |  |  |  |  |  |
| Inhalation |  |  |  |  | 37,2 mg/m3 |  |  |  | 308 mg/m3 |  |
| Skin |  |  |  |  | 121 mg/kg bw/d |  |  |  | 283 mg/kg bw/d |  |
| galaxolide |  |  |  |  |  |  |  |  |  |  |
| Predicted no-effect concentration - PNEC |  |  |  |  |  |  |  |  |  |  |
| Normal value in fresh water |  |  |  |  | 0,0044 | mg/l |  |  |  |  |
| Normal value in marine water |  |  |  |  | 0,00044 | mg/l |  |  |  |  |
| Normal value for fresh water sediment |  |  |  |  | 2 | mg/kg/d |  |  |  |  |
| Normal value for marine water sediment |  |  |  |  | 0,394 | mg/kg/d |  |  |  |  |
| Normal value for water, intermittent release |  |  |  |  | 0,03 | mg/l |  |  |  |  |
| Normal value of STP microorganisms |  |  |  |  | 1 | mg/l |  |  |  |  |
| Normal value for the food chain (secondary poisoning) |  |  |  |  | 3,3 | mg/kg food |  |  |  |  |
| Normal value for the terrestrial compartment |  |  |  |  | 0,31 | mg/kg/d |  |  |  |  |
| Health - Derived no-effect level - DNEL / DMEL |  |  |  |  |  |  |  |  |  |  |
|  | Effects on consumers |  |  |  |  | Effects on workers |  |  |  |  |
| Route of exposure | Acute local |  | Acute systemic | Chronic local | Chronic systemic | Acute local | Acute systemic | Chronic local | Chronic systemic |  |
| Oral |  |  |  |  | 3,8 mg/kg |  |  |  |  |  |

| Suarez Company |  |  |  |  |  |  | Revision nr. 2 Dated 16/01/2025 Printed on 16/01/2025 10/31 Replaced revision:1 (Dated: 04/09/2023) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO |  |  |  |  |  |  |  |  |  |
| bw/d |  |  |  |  |  |  |  |  |  |
| Inhalation |  |  |  | 6,5 mg/m3 |  |  |  | 22 mg/m3 |  |
| Skin |  |  |  | 36 mg/kg bw/d |  |  |  | 60 mg/kg bw/d |  |
| LINALYL ACETATE |  |  |  |  |  |  |  |  |  |
| Predicted no-effect concentration - PNEC |  |  |  |  |  |  |  |  |  |
| Normal value in fresh water |  |  |  | 0,011 | mg/l |  |  |  |  |
| Normal value in marine water |  |  |  | 0,001 | mg/l |  |  |  |  |
| Normal value for fresh water sediment |  |  |  | 0,609 | mg/kg |  |  |  |  |
| Normal value for marine water sediment |  |  |  | 0,061 | mg/kg |  |  |  |  |
| Normal value for water, intermittent release |  |  |  | 0,11 | mg/l |  |  |  |  |
| Normal value of STP microorganisms |  |  |  | 10 | mg/l |  |  |  |  |
| Normal value for the terrestrial compartment |  |  |  | 0,115 | mg/kg |  |  |  |  |
| Health - Derived no-effect level - DNEL / DMEL |  |  |  |  |  |  |  |  |  |
|  | Effects on consumers |  |  |  | Effects on workers |  |  |  |  |
| Route of exposure | Acute local | Acute systemic | Chronic local | Chronic systemic | Acute local | Acute systemic | Chronic local | Chronic systemic |  |
| Oral |  |  |  | 0,2 mg/kg bw/d |  |  |  |  |  |
| Inhalation |  |  |  | 0,68 mg/m3 |  |  |  | 2,75 mg/m3 |  |
| Skin | 8 mg/cm2 |  | 8 mg/cm2 | 1,25 mg/kg bw/d |  |  | 8 mg/cm2 | 2,5 mg/kg bw/d |  |
| geraniol |  |  |  |  |  |  |  |  |  |
| Predicted no-effect concentration - PNEC |  |  |  |  |  |  |  |  |  |
| Normal value in fresh water |  |  |  | 0,011 | mg/l |  |  |  |  |
| Normal value in marine water |  |  |  | 0,001 | mg/l |  |  |  |  |
| Normal value for fresh water sediment |  |  |  | 0,115 | mg/kg |  |  |  |  |
| Normal value for marine water sediment |  |  |  | 0,011 | mg/kg |  |  |  |  |
| Normal value for water, intermittent release |  |  |  | 0,108 | mg/l |  |  |  |  |
| Normal value of STP microorganisms |  |  |  | 0,7 | mg/l |  |  |  |  |
| Normal value for the terrestrial compartment |  |  |  | 0,017 | mg/kg |  |  |  |  |
| Health - Derived no-effect level - DNEL / DMEL |  |  |  |  |  |  |  |  |  |
|  | Effects on consumers |  |  |  | Effects on workers |  |  |  |  |
| Route of exposure | Acute local | Acute systemic | Chronic local | Chronic systemic | Acute local | Acute systemic | Chronic local | Chronic systemic |  |
| Oral |  |  |  | 13,75 mg/kg bw/d |  |  |  |  |  |
| Inhalation |  |  | 11,8 | 47,8 mg/m3 |  |  |  | 161,6 mg/m3 |  |
| Skin |  |  | 11,8 mg/cm2 | 7,5 mg/kg bw/d |  |  | 11,8 mg/cm2 | 12,5 mg/kg bw/d |  |
| 2H-CHROMEN-2-ONE |  |  |  |  |  |  |  |  |  |
| Predicted no-effect concentration - PNEC |  |  |  |  |  |  |  |  |  |
| Normal value in fresh water |  |  |  | 0,019 | mg/l |  |  |  |  |
| Normal value in marine water |  |  |  | 0,0019 | mg/l |  |  |  |  |
| Normal value for fresh water sediment |  |  |  | 0,15 | mg/kg |  |  |  |  |
| Normal value for marine water sediment |  |  |  | 0,015 | mg/kg |  |  |  |  |

| Suarez Company |  |  |  |  |  |  | Revision nr. 2 Dated 16/01/2025 Printed on 16/01/2025 11/31 Replaced revision:1 (Dated: 04/09/2023) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO |  |  |  |  |  |  |  |  |  |  |
| Normal value for water, intermittent release |  |  |  | 0,0142 | mg/l |  |  |  |  |  |
| Normal value of STP microorganisms |  |  |  | 6,4 | mg/l |  |  |  |  |  |
| Normal value for the food chain (secondary poisoning) |  |  |  | 30,7 | mg/kg |  |  |  |  |  |
| Normal value for the terrestrial compartment |  |  |  | 0,018 | mg/kg |  |  |  |  |  |
| Health - Derived no-effect level - DNEL / DMEL |  |  |  |  |  |  |  |  |  |  |
|  | Effects on consumers |  |  |  | Effects on workers |  |  |  |  |  |
| Route of exposure | Acute local | Acute systemic | Chronic local | Chronic systemic | Acute local | Acute systemic |  | Chronic local | Chronic systemic |  |
| Oral |  |  |  | 0,39 mg/kg bw/d |  |  |  |  |  |  |
| Inhalation |  |  |  | 1,69 mg/m3 |  |  |  |  | 6,78 mg/m3 |  |
| Skin |  |  |  | 0,39 mg/kg bw/d |  |  |  |  | 0,79 mg/kg bw/d |  |
| 3,7-DIMETHYLNONA-1,6-DIEN-3-OL |  |  |  |  |  |  |  |  |  |  |
| Predicted no-effect concentration - PNEC |  |  |  |  |  |  |  |  |  |  |
| Normal value in fresh water |  |  |  | 0,023 | mg/l |  |  |  |  |  |
| Normal value in marine water |  |  |  | 0,002 | mg/l |  |  |  |  |  |
| Normal value for fresh water sediment |  |  |  | 0,223 | mg/kg |  |  |  |  |  |
| Normal value for marine water sediment |  |  |  | 0,022 | mg/kg |  |  |  |  |  |
| Normal value for water, intermittent release |  |  |  | 0,23 | mg/l |  |  |  |  |  |
| Normal value of STP microorganisms |  |  |  | 10 | mg/l |  |  |  |  |  |
| Normal value for the food chain (secondary poisoning) |  |  |  | 8,53 | mg/kg |  |  |  |  |  |
| Normal value for the terrestrial compartment |  |  |  | 0,031 | mg/kg |  |  |  |  |  |
| Health - Derived no-effect level - DNEL / DMEL |  |  |  |  |  |  |  |  |  |  |
|  | Effects on consumers |  |  |  | Effects on workers |  |  |  |  |  |
| Route of exposure | Acute local | Acute systemic | Chronic local | Chronic systemic | Acute local | Acute systemic |  | Chronic local | Chronic systemic |  |
| Oral |  | 1,3 mg/kg bw/d |  | 0,2 mg/kg bw/d |  |  |  |  |  |  |
| Inhalation |  | 4,4 mg/m3 |  | 0,74 mg/m3 |  |  |  |  | 3 mg/m3 |  |
| Skin | 16 mg/cm2 | 2,7 mg/kg bw/d | 16 mg/cm2 | 1,4 mg/kg bw/d | 16 mg/cm2 | 5,5 mg/kg bw/d |  | 16 mg/cm2 | 2,7 mg/kg bw/d |  |
| 3,7-DIMETHYLOCT-6-EN-1-OL |  |  |  |  |  |  |  |  |  |  |
| Predicted no-effect concentration - PNEC |  |  |  |  |  |  |  |  |  |  |
| Normal value in fresh water |  |  |  | 0,002 | mg/l |  |  |  |  |  |
| Normal value in marine water |  |  |  | 0 | mg/l |  |  |  |  |  |
| Normal value for fresh water sediment |  |  |  | 0,026 | mg/kg |  |  |  |  |  |
| Normal value for marine water sediment |  |  |  | 0,003 | mg/kg |  |  |  |  |  |
| Normal value for water, intermittent release |  |  |  | 24 | mg/l |  |  |  |  |  |
| Normal value of STP microorganisms |  |  |  | 580 | mg/l |  |  |  |  |  |
| Normal value for the terrestrial compartment |  |  |  | 0,004 | mg/kg |  |  |  |  |  |
| Health - Derived no-effect level - DNEL / DMEL |  |  |  |  |  |  |  |  |  |  |
|  | Effects on consumers |  |  |  | Effects on workers |  |  |  |  |  |
| Route of exposure | Acute local | Acute systemic | Chronic local | Chronic systemic | Acute local | Acute systemic |  | Chronic local | Chronic systemic |  |
| Oral |  |  | 13,8 mg/kg bw/d |  |  |  |  |  |  |  |
| Inhalation | 10 mg/m3 |  | 10 mg/m3 | 47,8 mg/m3 |  |  |  | 10 mg/m3 1h | 161,6 mg/m3 |  |

| Suarez Company |  |  |  |  |  |  |  | Revision nr. 2 Dated 16/01/2025 Printed on 16/01/2025 12/31 Replaced revision:1 (Dated: 04/09/2023) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO |  |  |  |  |  |  |  |  |  |  |  |
| Skin |  | 2,95 mg/cm2 |  |  | 196,4 mg/kg | 2,95 mg/cm2 |  |  |  | 327,4 mg/kg |  |
|  |  |  |  |  | bw/d |  |  |  |  | bw/d |  |
| METHANOL |  |  |  |  |  |  |  |  |  |  |  |
| Threshold Limit Value |  |  |  |  |  |  |  |  |  |  |  |
| Type | Country | TWA/8h |  |  | STEL/15min | Remarks / |  |  |  |  |  |
|  |  |  |  |  |  | Observations |  |  |  |  |  |
| mg/m3 |  |  |  | ppm | mg/m3 | ppm |  |  |  |  |  |
| TLV | BGR | 50 |  |  |  | SKIN |  |  |  |  |  |
| TLV | CZE | 250 |  |  | 1000 | SKIN |  |  |  |  |  |
| AGW | DEU | 270 |  | 200 | 1080 | 800 | SKIN |  |  |  |  |
| MAK | DEU | 270 |  | 200 | 1080 | 800 | SKIN |  |  |  |  |
| TLV | DNK | 260 |  | 200 |  |  |  |  |  |  |  |
| VLA | ESP | 266 |  | 200 |  | SKIN |  |  |  |  |  |
| VLEP | FRA | 260 |  | 200 | 1300 | 1000 | SKIN |  |  |  |  |
| HTP | FIN | 270 |  | 200 | 330 | 250 | SKIN |  |  |  |  |
| TLV | GRC | 260 |  | 200 | 325 | 250 |  |  |  |  |  |
| AK | HUN | 260 |  |  | 1040 |  |  |  |  |  |  |
| GVI/KGVI | HRV | 260 |  | 200 |  | SKIN |  |  |  |  |  |
| VLEP | ITA | 260 |  | 200 |  | SKIN |  |  |  |  |  |
| TLV | NOR | 130 |  | 100 |  | SKIN |  |  |  |  |  |
| TGG | NLD | 133 |  | 100 |  | SKIN |  |  |  |  |  |
| VLE | PRT | 260 |  | 200 |  | SKIN |  |  |  |  |  |
| NDS/NDSCh | POL | 100 |  |  | 300 |  |  |  |  |  |  |
| NGV/KGV | SWE | 250 |  | 200 | 350 | 250 | SKIN |  |  |  |  |
| NPEL | SVK | 260 |  | 200 |  | SKIN |  |  |  |  |  |
| WEL | GBR | 266 |  | 200 | 333 | 250 | SKIN |  |  |  |  |
| OEL | EU | 260 |  | 200 |  | SKIN |  |  |  |  |  |
| TLV-ACGIH |  | 262 |  | 200 | 328 | 250 |  |  |  |  |  |
| Predicted no-effect concentration - PNEC |  |  |  |  |  |  |  |  |  |  |  |
| Normal value in fresh water |  |  |  |  | 20,8 | mg/l |  |  |  |  |  |
| Normal value in marine water |  |  |  |  | 2,08 | mg/l |  |  |  |  |  |
| Normal value for fresh water sediment |  |  |  |  | 77 | mg/kg |  |  |  |  |  |
| Normal value for marine water sediment |  |  |  |  | 7,7 | mg/kg |  |  |  |  |  |
| Normal value for water, intermittent release |  |  |  |  | 1540 | mg/l |  |  |  |  |  |
| Normal value of STP microorganisms |  |  |  |  | 100 | mg/l |  |  |  |  |  |
| Normal value for the terrestrial compartment |  |  |  |  | 100 | mg/kg |  |  |  |  |  |
| Health - Derived no-effect level - DNEL / DMEL |  |  |  |  |  |  |  |  |  |  |  |
| Effects on |  |  |  |  |  | Effects on |  |  |  |  |  |
| consumers |  |  |  |  |  | workers |  |  |  |  |  |
| Route of exposure |  | Acute local | Acute systemic | Chronic local | Chronic | Acute local | Acute |  | Chronic local | Chronic |  |
|  |  |  |  |  | systemic | systemic |  |  |  | systemic |  |
| Oral |  |  | 4 mg/kg bw/d |  | 4 mg/kg bw/d |  |  |  |  |  |  |
| Inhalation |  | 26 mg/m3 | 26 mg/m3 | 26 mg/m3 | 26 mg/m3 | 130 mg/m3 | 130 mg/m3 |  | 130 mg/m3 | 130 mg/m3 |  |
| Skin |  |  | 4 mg/kg bw/d |  | 4 mg/kg bw/d | 20 mg/kg bw/d |  |  |  | 20 mg/kg bw/d |  |
| toluene |  |  |  |  |  |  |  |  |  |  |  |

| Suarez Company |  |  |  |  |  |  |  | Revision nr. 2 Dated 16/01/2025 Printed on 16/01/2025 13/31 Replaced revision:1 (Dated: 04/09/2023) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO |  |  |  |  |  |  |  |  |  |  |  |
| Threshold Limit Value |  |  |  |  |  |  |  |  |  |  |  |
| Type | Country | TWA/8h |  |  | STEL/15min |  | Remarks / Observations |  |  |  |  |
|  |  | mg/m3 |  | ppm | mg/m3 | ppm |  |  |  |  |  |
| TLV | BGR | 150 |  |  | 300 |  |  |  |  |  |  |
| TLV | CZE | 200 |  |  | 500 |  | SKIN |  |  |  |  |
| AGW | DEU | 190 |  | 50 | 760 | 200 | SKIN |  |  |  |  |
| MAK | DEU | 190 |  | 50 | 760 | 200 |  |  |  |  |  |
| TLV | DNK | 94 |  | 25 |  |  | SKIN |  |  |  |  |
| VLA | ESP | 192 |  | 50 | 384 | 100 | SKIN |  |  |  |  |
| VLEP | FRA | 76,8 |  | 20 | 384 | 100 | SKIN |  |  |  |  |
| HTP | FIN | 81 |  | 25 | 380 | 100 | SKIN |  |  |  |  |
| TLV | GRC | 192 |  | 50 | 384 | 100 |  |  |  |  |  |
| AK | HUN | 190 |  |  | 760 |  |  |  |  |  |  |
| GVI/KGVI | HRV | 192 |  | 50 | 384 | 100 | SKIN |  |  |  |  |
| VLEP | ITA | 192 |  | 50 |  |  | SKIN |  |  |  |  |
| TLV | NOR | 94 |  | 25 |  |  | SKIN |  |  |  |  |
| TGG | NLD | 150 |  |  | 384 |  |  |  |  |  |  |
| VLE | PRT | 192 |  | 50 | 384 | 100 | SKIN |  |  |  |  |
| NDS/NDSCh | POL | 100 |  |  | 200 |  |  |  |  |  |  |
| TLV | ROU | 192 |  | 50 | 384 | 100 | SKIN |  |  |  |  |
| NGV/KGV | SWE | 192 |  | 50 | 384 | 100 | SKIN |  |  |  |  |
| NPEL | SVK | 192 |  | 50 | 384 |  | SKIN |  |  |  |  |
| MV | SVN | 192 |  | 50 | 384 | 100 | SKIN |  |  |  |  |
| WEL | GBR | 191 |  | 50 | 384 | 100 | SKIN |  |  |  |  |
| OEL | EU | 192 |  | 50 | 384 | 100 | SKIN |  |  |  |  |
| TLV-ACGIH |  | 75,4 |  | 20 |  |  |  |  |  |  |  |
| Predicted no-effect concentration - PNEC |  |  |  |  |  |  |  |  |  |  |  |
| Normal value in fresh water |  |  |  |  | 0,68 | mg/l |  |  |  |  |  |
| Normal value in marine water |  |  |  |  | 0,68 | mg/l |  |  |  |  |  |
| Normal value for fresh water sediment |  |  |  |  | 16,39 | mg/kg/d |  |  |  |  |  |
| Normal value for marine water sediment |  |  |  |  | 16,39 | mg/kg/d |  |  |  |  |  |
| Normal value for water, intermittent release |  |  |  |  | 0,68 | mg/l |  |  |  |  |  |
| Normal value of STP microorganisms |  |  |  |  | 13,61 | mg/l |  |  |  |  |  |
| Normal value for the food chain (secondary poisoning) |  |  |  |  | 2,89 | mg/kg soil dw |  |  |  |  |  |
| Health - Derived no-effect level - DNEL / DMEL |  |  |  |  |  |  |  |  |  |  |  |
|  | Effects on consumers |  |  |  |  | Effects on workers |  |  |  |  |  |
| Route of exposure | Acute local |  | Acute systemic | Chronic local | Chronic systemic | Acute local | Acute systemic |  | Chronic local | Chronic systemic |  |
| Oral |  |  |  |  | 8,13 mg/kg bw/d |  |  |  |  |  |  |
| Inhalation | 226 mg/m3 |  | 226 mg/m3 | 56,5 mg/m3 | 56,5 mg/m3 | 384 mg/m3 | 384 mg/m3 |  | 192 mg/m3 | 192 mg/m3 |  |
| Skin |  |  |  |  | 226 mg/kg bw/d |  |  |  |  | 384 mg/kg bw/d |  |
| ACETONE |  |  |  |  |  |  |  |  |  |  |  |
| Threshold Limit Value |  |  |  |  |  |  |  |  |  |  |  |

| BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO |  |  |  |  |  |  | Printed on 16/01/2025 14/31 Replaced revision:1 (Dated: 04/09/2023) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type | Country | TWA/8h |  | STEL/15min | Remarks / Observations |  |  |  |  |
|  |  | mg/m3 | ppm | mg/m3 | ppm |  |  |  |  |
| TLV | BGR | 600 |  | 1400 |  |  |  |  |  |
| TLV | CZE | 800 | 331,2 | 1500 | 621 |  |  |  |  |
| AGW | DEU | 1200 | 500 | 2400 (C) | 1000 (C) |  |  |  |  |
| MAK | DEU | 1200 | 500 | 2400 | 1000 |  |  |  |  |
| TLV | DNK | 600 | 250 |  |  |  | E |  |  |
| VLEP | FRA | 1210 | 500 | 2420 | 1000 |  |  |  |  |
| HTP | FIN | 1200 | 500 | 1500 | 630 |  |  |  |  |
| TLV | GRC | 1780 |  | 3560 |  |  |  |  |  |
| AK | HUN | 1210 |  |  |  |  |  |  |  |
| GVI/KGVI | HRV | 1210 | 500 |  |  |  |  |  |  |
| VLEP | ITA | 1210 | 500 |  |  |  |  |  |  |
| TLV | NOR | 295 | 125 |  |  |  |  |  |  |
| TGG | NLD | 1210 |  | 2420 |  |  |  |  |  |
| VLE | PRT | 1210 | 500 |  |  |  |  |  |  |
| NDS/NDSCh | POL | 600 |  | 1800 |  |  |  |  |  |
| TLV | ROU | 1210 | 500 |  |  |  |  |  |  |
| NGV/KGV | SWE | 600 | 250 | 1200 (C) | 500 (C) |  |  |  |  |
| NPEL | SVK | 1210 | 500 |  |  |  |  |  |  |
| MV | SVN | 1210 | 500 | 2420 | 1000 |  |  |  |  |
| WEL | GBR | 1210 | 500 | 3620 | 1500 |  |  |  |  |
| OEL | EU | 1210 | 500 |  |  |  |  |  |  |
| TLV-ACGIH |  |  | 250 |  | 500 |  |  |  |  |
| XYLENE |  |  |  |  |  |  |  |  |  |
| Threshold Limit Value |  |  |  |  |  |  |  |  |  |
| Type | Country | TWA/8h |  | STEL/15min |  | Remarks / Observations |  |  |  |
|  |  | mg/m3 | ppm | mg/m3 | ppm |  |  |  |  |
| TLV | BGR | 221 | 50 | 442 | 100 | SKIN |  |  |  |
| TLV | CZE | 200 | 45,4 | 400 | 90,8 | SKIN |  |  |  |
| AGW | DEU | 440 | 100 | 880 | 200 | SKIN |  |  |  |
| MAK | DEU | 440 | 100 | 880 | 200 | SKIN |  |  |  |
| TLV | DNK | 109 | 25 |  |  | SKIN |  | E |  |
| VLA | ESP | 221 | 50 | 442 | 100 | SKIN |  |  |  |
| VLEP | FRA | 221 | 50 | 442 | 100 | SKIN |  |  |  |
| HTP | FIN | 220 | 50 | 440 | 100 | SKIN |  |  |  |
| TLV | GRC | 435 | 100 | 650 | 150 |  |  |  |  |
| AK | HUN | 221 |  | 442 |  | SKIN |  |  |  |
| GVI/KGVI | HRV | 221 | 50 | 442 | 100 | SKIN |  |  |  |
| VLEP | ITA | 221 | 50 | 442 | 100 | SKIN |  |  |  |
| TLV | NOR | 108 | 25 |  |  | SKIN |  |  |  |

| BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO Printed on 16/01/2025 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | BIANCO |  |  |  |  | 15/31 |
|  |  |  |  |  |  |  |  | Replaced revision:1 (Dated: 04/09/2023) |
| TGG | NLD | 210 |  |  | 442 |  | SKIN |  |
| VLE | PRT | 221 |  | 50 | 442 | 100 | SKIN |  |
| NDS/NDSCh | POL | 100 |  |  | 200 |  | SKIN |  |
| TLV | ROU | 221 |  | 50 | 442 | 100 | SKIN |  |
| NGV/KGV | SWE | 221 |  | 50 | 442 | 100 | SKIN |  |
| NPEL | SVK | 221 |  | 50 | 442 | 100 | SKIN |  |
| MV | SVN | 221 |  | 50 | 442 | 100 | SKIN |  |
| WEL | GBR | 220 |  | 50 | 441 | 100 | SKIN |  |
| OEL | EU | 221 |  | 50 | 442 | 100 | SKIN |  |
| TLV-ACGIH |  |  |  | 20 |  |  |  |  |
| Legend: (C) = CEILING ; INHAL = Inhalable Fraction ; RESP = Respirable Fraction ; THORA = Thoracic Fraction. VND = hazard identified but no DNEL/PNEC available ; NEA = no exposure expected ; NPI = no hazard identified ; LOW = low hazard ; MED = medium hazard ; HIGH = high hazard. 8.2. Exposure controls As the use of adequate technical equipment must always take priority over personal protective equipment, make sure that the workplace is well aired through effective local aspiration. When choosing personal protective equipment, ask your chemical substance supplier for advice. Personal protective equipment must be CE marked, showing that it complies with applicable standards. Provide an emergency shower with face and eye wash station. HAND PROTECTION Protect hands with category III work gloves. The following should be considered when choosing work glove material (see standard EN 374): compatibility, degradation, permeability time. The work gloves' resistance to chemical agents should be checked before use, as it can be unpredictable. The gloves' wear time depends on the duration and type of use. SKIN PROTECTION Wear category II professional long-sleeved overalls and safety footwear (see Regulation 2016/425 and standard EN ISO 20344). Wash body with soap and water after removing protective clothing. Consider the appropriateness of providing antistatic clothing in the case of working environments in which there is a risk of explosion. EYE PROTECTION Wear airtight protective goggles (see standard EN ISO 16321). RESPIRATORY PROTECTION Respiratory protection devices must be used if the technical measures adopted are not suitable for restricting the worker's exposure to the threshold values considered. Use a mask with a type A filter whose class (1, 2 or 3) must be chosen according to the limit of use concentration. (see standard EN 14387). If the substance considered is odourless or its olfactory threshold is higher than the corresponding TLV-TWA and in the case of an emergency, wear open-circuit compressed air breathing apparatus (in compliance with standard EN 137) or external air-intake breathing apparatus (in compliance with standard EN 138). For a correct choice of respiratory protection device, see standard EN 529. |  |  |  |  |  |  |  |  |

# BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO

Printed on 16/01/2025

16/31

Replaced revision:1 (Dated: 04/09/2023)

### ENVIRONMENTAL EXPOSURE CONTROLS

The emissions generated by manufacturing processes, including those generated by ventilation equipment, should be checked to ensure compliance with environmental standards.

Product residues must not be indiscriminately disposed of with waste water or by dumping in waterways.

## SECTION 9. Physical and chemical properties

  - Information on basic physical and chemical properties

| Properties | Value | Information |
| --- | --- | --- |
| Appearance | liquid | Temperature: 20 °C |
| Colour | green |  |
| Odour | characteristic |  |
| Melting point / freezing point | -114 °C | Method:not specified |
|  |  | Remark:ACGIG, 2014; HSDB 2015; INRS, 2011 |
|  |  | Substance:ETHANOL |
| Initial boiling point | 78,5 °C | Method:not specified |
|  |  | Remark:ACGIH, 2014, at atmospheric pressure |
|  |  | Substance:ETHANOL |
|  |  | Initial boiling point: 78 °C |
| Flammability | flammable liquid | Method:ISO 3679:2023 |
| Lower explosive limit | 3,3 % (v/v) | Method:Not specified |
|  |  | Remark:INAIL, 2018 |
|  |  | Substance:ETHANOL |
|  |  | Temperature: 20 °C |
| Upper explosive limit | 19 % (v/v) | Method:Not specified |
|  |  | Remark:INAIL, 2018 |
|  |  | Substance:ETHANOL |
|  |  | Temperature: 20 °C |
| Flash point | 18,5 °C | Method:ISO 3679:2023 |
| Auto-ignition temperature | 363 °C | Method:not specified |
|  |  | Substance:ETHANOL |
| Decomposition temperature | not available | Reason for missing data:It only applies to authoritative substances and mixtures, organic peroxides and other substances and mixtures that they can decompose |
| pH | 5 - 8 | Method:not specified |
|  |  | Remark:64-17-5 10% in H20 |
|  |  | Temperature: 20 °C |
| Kinematic viscosity | 1,5 | Method:Not specified |
|  |  | Remark:64-17-5 |
|  |  | Temperature: 20 °C |
| Solubility | soluble in water | Method:not specified |
|  |  | Substance:ETHANOL |
|  |  | Temperature: 20 °C |
| Partition coefficient: n-octanol/water | -0,35 | Method:not specified |

# BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO

Printed on 16/01/2025

17/31

Replaced revision:1 (Dated: 04/09/2023)

|  |  | Substance:ETHANOL |
| --- | --- | --- |
|  |  | Temperature: 20 °C |
| Dispersion stability | not available | Reason for missing data:The mixture does not contain nanoform |
| Vapour pressure | 5,9 kPa | Method:not specified |
|  |  | Substance:ETHANOL |
|  |  | Vapour pressure: 45 mmHg |
|  |  | Temperature: 20 °C |
| Density and/or relative density | 0,855 g/cm3 | Method:not specified |
|  |  | Temperature: 20 °C |
| Relative vapour density | 1,6 | Method:HSDB, 2015; INRS, 2011 |
|  |  | Substance:ETHANOL |
|  |  | Temperature: 20 °C |

Particle characteristics Median equivalent diameter

| Remark: | It only applies to solids |
| --- | --- |

| Remark: | It only applies to solids |
| --- | --- |

| Remark: | It only applies to solids |
| --- | --- |

Size distribution Dustiness

| Remark: | It only applies to solids |
| --- | --- |

| Remark: | It only applies to solids |
| --- | --- |

Specific surface area Shape

  - Other information

    - Information with regard to physical hazard classes

Information not available

    - Other safety characteristics

| Explosive properties | not available | Reason for missing data:Absent chemical groups associated with explosive properties in accordance with the provisions of Annex I, Part 2, chap. 2.1.4.3 of Reg. (EC) 1272/2008 - CLP |
| --- | --- | --- |
| Oxidising properties | not available | Reason for missing data:Absent requirements related to the presence of atoms or chemical bonds associated with oxidizing properties in the molecules of the components according to Annex I, Part 2, 2.13.4 Reg. (CE) 1272/2008 |

## SECTION 10. Stability and reactivity

10.1. Reactivity

There are no particular risks of reaction with other substances in normal conditions of use.

| Suarez Company | Revision nr. 2 Dated 16/01/2025 Printed on 16/01/2025 18/31 Replaced revision:1 (Dated: 04/09/2023) |
| --- | --- |
| BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO |  |
| DIPROPYLENE GLYCOL MONOMETHYL ETHER May react with: oxidising substances.When heated to decomposition releases: harsh fumes,zinc alloys. toluene Avoid exposure to: light. ACETONE Decomposes under the effect of heat. - Chemical stability The product is stable in normal conditions of use and storage. - Possibility of hazardous reactions The vapours may also form explosive mixtures with the air. ETHANOL Risk of explosion on contact with: alkaline metals,alkaline oxides,calcium hypochlorite,sulphur monofluoride,acetic anhydride,acids,concentrated hydrogen peroxide,perchlorates,perchloric acid,perchloronitrile,mercury nitrate,nitric acid,silver,silver nitrate,ammonia,silver oxide,ammonia,strong oxidising agents,nitrogen dioxide.May react dangerously with: bromoacetylene,chlorine acetylene,bromine trifluoride,chromium trioxide,chromyl chloride,fluorine,potassium tert-butoxide,lithium hydride,phosphorus trioxide,black platinum,zirconium (IV) chloride,zirconium (IV) iodide.Forms explosive mixtures with: air. toluene Risk of explosion on contact with: fuming sulphuric acid,nitric acid,silver perchlorate,nitrogen dioxide,non-metal halogenates,acetic acid,organic nitrocompounds.May form explosive mixtures with: air.May react dangerously with: strong oxidising agents,strong acids,sulphur. ACETONE Risk of explosion on contact with: bromine trifluoride,fluorine dioxide,hydrogen peroxide,nitrosyl chloride,2-methyl-1,3 butadiene,nitromethane,nitrosyl perchlorate.May react dangerously with: potassium tert-butoxide,alkaline hydroxides,bromine,bromoform,isoprene,sodium,sulphur dioxide,chromium trioxide,chromyl chloride,nitric acid,chloroform,peroxymonosulphuric acid,phosphoryl oxychloride,chromosulphuric acid,fluorine,strong oxidising agents,strong reducing agents.Develops flammable gas on contact with: nitrosyl perchlorate. XYLENE Stable in normal conditions of use and storage.Reacts violently with: strong oxidants,strong acids,nitric acid,perchlorates.May form explosive mixtures with: air. - Conditions to avoid Avoid overheating. Avoid bunching of electrostatic charges. Avoid all sources of ignition. ETHANOL Avoid exposure to: sources of heat,naked flames. ACETONE |  |

| Suarez Company | Revision nr. 2 Dated 16/01/2025 Printed on 16/01/2025 19/31 Replaced revision:1 (Dated: 04/09/2023) |
| --- | --- |
| BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO |  |
| Avoid exposure to: sources of heat,naked flames. - Incompatible materials ACETONE Incompatible with: acids,oxidising substances. - Hazardous decomposition products In the event of thermal decomposition or fire, gases and vapours that are potentially dangerous to health may be released. ACETONE May develop: ketenes,irritant substances. SECTION 11. Toxicological information In the absence of experimental data for the product itself, health hazards are evaluated according to the properties of the substances it contains, using the criteria specified in the applicable regulation for classification. It is therefore necessary to take into account the concentration of the individual hazardous substances indicated in section 3, to evaluate the toxicological effects of exposure to the product. - Information on hazard classes as defined in Regulation (EC) No 1272/2008 Metabolism, toxicokinetics, mechanism of action and other information Information not available Information on likely routes of exposure METHANOL WORKERS: inhalation; contact with the skin. POPULATION: ingestion of contaminated food or water; contact with the skin of products containing the substance. toluene WORKERS: inhalation; contact with the skin. POPULATION: ingestion of contaminated food or water; inhalation of ambient air; contact with the skin of products containing the substance. XYLENE WORKERS: inhalation; contact with the skin. POPULATION: ingestion of contaminated food or water; inhalation of ambient air. Delayed and immediate effects as well as chronic effects from short and long-term exposure METHANOL The minimum lethal dose for humans by ingestion is considered to be in the range from 300 to 1000 mg/kg. Ingestion of 4-10 ml of the substance may cause permanent blindness in adult humans (IPCS). toluene Toxic effect on the central and peripheral nervous system with encephalopathy and polyneuritis; irritating for the skin, conjunctiva, cornea and respiratory apparatus. |  |

| Suarez Company | Revision nr. 2 Dated 16/01/2025 Printed on 16/01/2025 20/31 Replaced revision:1 (Dated: 04/09/2023) |
| --- | --- |
| BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO |  |
| XYLENE Toxic effect on the central nervous system (encephalopathy); irritating for the skin, conjunctiva, cornea and respiratory apparatus. Interactive effects toluene Certain drugs and other industrial products can interfere with the metabolism of the toluene. XYLENE Intake of alcohol interferes with the metabolism of the substance, inhibiting it. Ethanol consumption (0.8 g/kg) before a 4-hour exposure to xylene vapours (145 and 280 ppm) causes a 50% reduction in the excretion of methyl hippuric acid, whereas the concentration of xylenes in the blood increases approx. 1.5-2 times. At the same time there is an increase in the secondary side effects of the ethanol. The metabolism of the xylenes is increased by phenobarbital and 3-methyl-colantrene type enzyme inducers. Aspirin and xylenes mutually inhibit their conjugation with the glycine, which results in a decrease in urinary excretion of methyl hippuric acid. Other industrial products can interfere with the metabolism of xylenes. ACUTE TOXICITY ATE (Inhalation) of the mixture: Not classified (no significant component) ATE (Oral) of the mixture: Not classified (no significant component) ATE (Dermal) of the mixture: Not classified (no significant component) ETHANOL LD50 (Oral): > 5000 mg/kg Rat LC50 (Inhalation vapours): 120 mg/l/4h Pimephales promelas DIPROPYLENE GLYCOL MONOMETHYL ETHER LD50 (Dermal): 10 mL/kg rabbit LD50 (Oral): > 5000 mg/kg rat LC50 (Inhalation vapours): 3,35 mg/L 7h ALPHA-ISOMETHYL IONONE LD50 (Dermal): > 2000 mg/kg rat LD50 (Oral): 1000 mg/kg mouse galaxolide LD50 (Dermal): > 5000 mg/kg rabbit LD50 (Oral): > 4640 mg/kg rat LINALYL ACETATE LD50 (Dermal): > 5000 mg/kg rabbit LD50 (Oral): > 9000 mg/kg rat geraniol LD50 (Dermal): > 5000 mg/kg rabbit LD50 (Oral): 3600 mg/kg bw rat 2H-CHROMEN-2-ONE LD50 (Dermal): 293 mg/kg bw rat LD50 (Oral): 290 mg/kg bw rat LC50 (Inhalation mists/powders): 293 mg/kg rat 3,7-DIMETHYLNONA-1,6-DIEN-3-OL LD50 (Dermal): > 5000 mg/kg rabbit LD50 (Oral): 2790 mg/kg rat LC50 (Inhalation vapours): > 1 mg/l rat METHANOL LD50 (Oral): > 1187 mg/kg Rat LC50 (Inhalation vapours): 43,68 mg/l/6h Rat |  |

# BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO

Printed on 16/01/2025

21/31

Replaced revision:1 (Dated: 04/09/2023)

toluene

| LD50 (Dermal): | > 5000 mg/kg bw Rabbit |
| --- | --- |
| LD50 (Oral): | > 5000 mg/kg bw Rat |
| LC50 (Inhalation vapours): | 100 ppm human |

### XYLENE

| LD50 (Dermal): | 4350 mg/kg Rabbit |
| --- | --- |
| ATE (Dermal): | 1100 mg/kg estimate from table 3.1.2 of Annex I of the CLP |
|  | (figure used for calculation of the acute toxicity estimate of the mixture) |
| LD50 (Oral): | 3523 mg/kg Rat |
| LC50 (Inhalation vapours): | 26 mg/l/4h Rat |

SKIN CORROSION / IRRITATION

Does not meet the classification criteria for this hazard class SERIOUS EYE DAMAGE / IRRITATION

Causes serious eye irritation RESPIRATORY OR SKIN SENSITISATION

Sensitising for the skin

### GERM CELL MUTAGENICITY

Does not meet the classification criteria for this hazard class CARCINOGENICITY

Does not meet the classification criteria for this hazard class

toluene

Classified in Group 3 (not classifiable as a human carcinogen) by the International Agency for Research on Cancer (IARC) - (IARC, 1999). The US Environmental Protection Agency (EPA) affirms that "the data is inadequate for an assessment of the carcinogenic potential".

### XYLENE

Classified in Group 3 (not classifiable as a human carcinogen) by the International Agency for Research on Cancer (IARC).

The US Environmental Protection Agency (EPA) affirms that "the data is inadequate for an assessment of the carcinogenic potential". REPRODUCTIVE TOXICITY

Does not meet the classification criteria for this hazard class STOT - SINGLE EXPOSURE

Does not meet the classification criteria for this hazard class STOT - REPEATED EXPOSURE

Does not meet the classification criteria for this hazard class ASPIRATION HAZARD

# BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO

Printed on 16/01/2025

22/31

Replaced revision:1 (Dated: 04/09/2023)

Does not meet the classification criteria for this hazard class

11.2. Information on other hazards

Based on the available data, the product does not contain substances listed in the main European lists of potential or suspected endocrine disruptors with human health effects under evaluation.

## SECTION 12. Ecological information

This product is dangerous for the environment and the aquatic organisms. In the long term, it has negative effects on the aquatic environment.

  - Toxicity

| 3,7-DIMETHYLNONA-1,6-DIEN-3-OL |  |
| --- | --- |
| LC50 - for Fish | 24 mg/l/96h |
| EC50 - for Crustacea | 23 mg/l/48h |
| EC50 - for Algae / Aquatic Plants | 13,3 mg/l/72h |
| Chronic NOEC for Fish | 5 mg/l |
| Chronic NOEC for Crustacea | 3,2 mg/l 48h |
| Chronic NOEC for Algae / Aquatic Plants | 6,3 mg/l |

| 3,7-DIMETHYLOCT-6-EN-1-OL |  |
| --- | --- |
| LC50 - for Fish | 14,66 mg/l/96h Leuciscus idus |
| EC50 - for Crustacea | 17,48 mg/l/48h Daphnia magna |
| EC50 - for Algae / Aquatic Plants | 2,4 mg/l/72h |
| Chronic NOEC for Fish | 4,6 mg/l Leuciscus idus |
| Chronic NOEC for Crustacea | 3,1 mg/l Daphnia magna |

| geraniol |  |
| --- | --- |
| LC50 - for Fish | 22 mg/l/96h Danio rerio |
| EC50 - for Crustacea | 10,5 mg/l/48h Daphnia magna |
| EC50 - for Algae / Aquatic Plants | 13,1 mg/l/72h Desmodesmus subspicatus |
| Chronic NOEC for Fish | 10 mg/l Dania rerio |

| toluene |  |
| --- | --- |
| LC50 - for Fish | 5,5 mg/l/96h Oncorhynchus kisutch |
| EC50 - for Crustacea | 3,78 mg/l |
| EC50 - for Algae / Aquatic Plants | 134 mg/l/3 h Chlorella vulgaris and Chlamydomonas angulosa |
| Chronic NOEC for Algae / Aquatic Plants | 10 mg/l/72 h Skeletonema costatum |

| LINALYL ACETATE |  |
| --- | --- |
| LC50 - for Fish | 11 mg/l/96h |
| EC50 - for Crustacea | 15 mg/l/48h |
| EC50 - for Algae / Aquatic Plants | 62 mg/l/72h |
| Chronic NOEC for Fish | 10 mg/l |
| Chronic NOEC for Crustacea | 10 mg/l |

# BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO

Printed on 16/01/2025

23/31

Replaced revision:1 (Dated: 04/09/2023)

| Chronic NOEC for Algae / Aquatic Plants | 9,6 mg/l |
| --- | --- |

| galaxolide |  |
| --- | --- |
| LC50 - for Fish | 1,36 mg/l/96h Lepomis macrochirus |
| EC50 - for Crustacea | 0,3 mg/l/48h Daphnia magna |
| EC50 - for Algae / Aquatic Plants | 0,723 mg/l/72h Pseudokirchneriella subcapitata |
| Chronic NOEC for Fish | 0,093 mg/l/21 d Lepomis macrochirus |
| Chronic NOEC for Crustacea | 0,038 mg/l Acartia tonsa |
| Chronic NOEC for Algae / Aquatic Plants | 0,201 mg/l/72 h Pseudokirchneriella subcapitata |

| ALPHA-ISOMETHYL IONONE |  |
| --- | --- |
| LC50 - for Fish | 5,495 mg/l/96h |
| EC50 - for Crustacea | 1,45 mg/l/48h |
| EC50 - for Algae / Aquatic Plants | 6,468 mg/l/72h |

| DIPROPYLENE GLYCOL MONOMETHYL ETHER |  |
| --- | --- |
| LC50 - for Fish | > 150 mg/l/96h |
| EC50 - for Crustacea | > 1000 mg/l/48h |
| EC50 - for Algae / Aquatic Plants | > 969 mg/l/72h |
| Chronic NOEC for Crustacea | 1000 mg/l 48h |
| Chronic NOEC for Algae / Aquatic Plants | 969 mg/l |

| ETHANOL |  |
| --- | --- |
| LC50 - for Fish | 14200 mg/l/96h |
| EC50 - for Crustacea | 454 mg/l/48h |
| EC50 - for Algae / Aquatic Plants | 275 mg/l/72h |
| Chronic NOEC for Fish | 250 mg/l |
| Chronic NOEC for Crustacea | 96 mg/l |
| Chronic NOEC for Algae / Aquatic Plants | 11,5 mg/l |

| METHANOL |  |
| --- | --- |
| LC50 - for Fish | 15320 mg/l/96h Leuciscus idus melanotus |
| EC50 - for Crustacea | 18260 mg/l/96 h Daphnia magna |
| EC50 - for Algae / Aquatic Plants | 22000 mg/l/96 h Pseudokirchneriella subcapitata |
| Chronic NOEC for Fish | 3950 mg/l/96 h Danio rerio |
| Chronic NOEC for Crustacea | 7960 mg/l/96 h Mytilus edulis |

| 2H-CHROMEN-2-ONE |  |
| --- | --- |
| LC50 - for Fish | 1,324 mg/l/96h |
| EC50 - for Crustacea | 8,012 mg/l/48h |
| EC50 - for Algae / Aquatic Plants | 1,452 mg/l/72h |
| Chronic NOEC for Fish | 0,119 mg/l |
| Chronic NOEC for Crustacea | 0,448 mg/l |

# BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO

Printed on 16/01/2025

24/31

Replaced revision:1 (Dated: 04/09/2023)

| Chronic NOEC for Algae / Aquatic Plants | 0,408 mg/l |
| --- | --- |

  - Persistence and degradability

### 3,7-DIMETHYLNONA-1,6-DIEN-3-OL

Rapidly degradable

### 3,7-DIMETHYLOCT-6-EN-1-OL

Rapidly degradable geraniol

Rapidly degradable

| toluene |  |
| --- | --- |
| Solubility in water | 100 - 1000 mg/l |

Rapidly degradable LINALYL ACETATE

Rapidly degradable galaxolide

Rapidly degradable

### ALPHA-ISOMETHYL IONONE

Entirely degradable

| DIPROPYLENE GLYCOL MONOMETHYL ETHER |  |
| --- | --- |
| Solubility in water | 1000 - 10000 mg/l |

Rapidly degradable

| ETHANOL |  |
| --- | --- |
| Solubility in water | 1000 - 10000 mg/l |

Rapidly degradable

| METHANOL |  |
| --- | --- |
| Solubility in water | 1000 - 10000 mg/l |

Rapidly degradable ACETONE

Rapidly degradable 2H-CHROMEN-2-ONE

Rapidly degradable

| XYLENE |  |
| --- | --- |
| Solubility in water | 100 - 1000 mg/l |

Rapidly degradable

  - Bioaccumulative potential

| toluene |  |
| --- | --- |
| Partition coefficient: n-octanol/water | 2,73 |
| BCF | 90 |

| LINALYL ACETATE |  |
| --- | --- |
| BCF | 173,9 l/kg |

| galaxolide |  |
| --- | --- |
| Partition coefficient: n-octanol/water | 5,3 |
| BCF | 498 5 d |

# BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO

Printed on 16/01/2025

25/31

Replaced revision:1 (Dated: 04/09/2023)

| ALPHA-ISOMETHYL IONONE |  |
| --- | --- |
| BCF | 726,4 l/kg |

| DIPROPYLENE GLYCOL MONOMETHYL ETHER |  |
| --- | --- |
| Partition coefficient: n-octanol/water | 0,0043 |

| ETHANOL |  |
| --- | --- |
| Partition coefficient: n-octanol/water | -0,35 |

| METHANOL |  |
| --- | --- |
| Partition coefficient: n-octanol/water | -0,77 |
| BCF | 0,2 |

| ACETONE |  |
| --- | --- |
| Partition coefficient: n-octanol/water | -0,23 |
| BCF | 3 |

| XYLENE |  |
| --- | --- |
| Partition coefficient: n-octanol/water | 3,12 |
| BCF | 25,9 |

  - Mobility in soil

| geraniol |  |
| --- | --- |
| Partition coefficient: soil/water | 1,85 |

| toluene |  |
| --- | --- |
| Partition coefficient: soil/water | 2,31 |

| LINALYL ACETATE |  |
| --- | --- |
| Partition coefficient: soil/water | 2,71 |

| galaxolide |  |
| --- | --- |
| Partition coefficient: soil/water | 4,39 |

| ALPHA-ISOMETHYL IONONE |  |
| --- | --- |
| Partition coefficient: soil/water | 3,038 a 20°C |

| XYLENE |  |
| --- | --- |
| Partition coefficient: soil/water | 2,73 |

  - Results of PBT and vPvB assessment

On the basis of available data, the product does not contain any PBT or vPvB in percentage ≥ than 0,1%.

# BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO

Printed on 16/01/2025

26/31

Replaced revision:1 (Dated: 04/09/2023)

  - Endocrine disrupting properties

Based on the available data, the product does not contain substances listed in the main European lists of potential or suspected endocrine disruptors with environmental effects under evaluation.

  - Other adverse effects

Information not available

## SECTION 13. Disposal considerations

  - Waste treatment methods

Reuse, when possible. Product residues should be considered special hazardous waste. The hazard level of waste containing this product should be evaluated according to applicable regulations.

Disposal must be performed through an authorised waste management firm, in compliance with national and local regulations. Waste transportation may be subject to ADR restrictions.

The management of waste arising from the use or dispersal of this product must be organised in accordance with occupational safety regulations. See section 8 for possible need for PPE.

### CONTAMINATED PACKAGING

Contaminated packaging must be recovered or disposed of in compliance with national waste management regulations.

## SECTION 14. Transport information

  - UN number or ID number

| ADR / RID, IMDG, IATA: | UN 1170 |
| --- | --- |

  - UN proper shipping name

| ADR / RID: | ETHANOL SOLUTION (ETHYL ALCOHOL SOLUTION) |
| --- | --- |
| IMDG: | ETHANOL SOLUTION (ETHYL ALCOHOL SOLUTION) |
| IATA: | ETHANOL SOLUTION (ETHYL ALCOHOL SOLUTION) |

  - Transport hazard class(es)

| ADR / RID: | Class: 3 | Label: 3 |  |
| --- | --- | --- | --- |
| IMDG: | Class: 3 | Label: 3 |  |
| IATA: | Class: 3 | Label: 3 |  |

  - Packing group

| ADR / RID, IMDG, IATA: | II |
| --- | --- |

# BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO

Printed on 16/01/2025

27/31

Replaced revision:1 (Dated: 04/09/2023)

  - Environmental hazards

| ADR / RID: | NO |
| --- | --- |
| IMDG: | not marine pollutant |
| IATA: | NO |

  - Special precautions for user

| ADR / RID: | HIN - Kemler: 33 | Limited Quantities: 1 lt | Tunnel restriction code: (D/E) |
| --- | --- | --- | --- |
|  | Special provision: 144, 601 |  |  |
| IMDG: | EMS: F-E, S-D | Limited Quantities: 1 lt |  |
| IATA: | Cargo: | Maximum quantity: 60 L | Packaging instructions: 364 |
|  | Passengers: | Maximum quantity: 5 L | Packaging instructions: 353 |
|  | Special provision: | A3, A58, A180 |  |

  - Maritime transport in bulk according to IMO instruments

Information not relevant

## SECTION 15. Regulatory information

  - Safety, health and environmental regulations/legislation specific for the substance or mixture

Seveso Category - Directive 2012/18/EU: P5c

Restrictions relating to the product or contained substances pursuant to Annex XVII to EC Regulation 1907/2006

Product

| Point | 3 - 40 |
| --- | --- |

Contained substance

| Point | 75 |
| --- | --- |

Regulation (EU) 2019/1148 - on the marketing and use of explosives precursors

not applicable

Substances in Candidate List (Art. 59 REACH)

On the basis of available data, the product does not contain any SVHC in percentage ≥ than 0,1%.

# BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO

Printed on 16/01/2025

28/31

Replaced revision:1 (Dated: 04/09/2023)

Substances subject to authorisation (Annex XIV REACH)

None

Substances subject to exportation reporting pursuant to Regulation (EU) 649/2012:

None

Substances subject to the Rotterdam Convention:

None

Substances subject to the Stockholm Convention:

None

Healthcare controls

Workers exposed to this chemical agent must not undergo health checks, provided that available risk-assessment data prove that the risks related to the workers' health and safety are modest and that the 98/24/EC directive is respected.

German regulation on the classification of substances hazardous to water (AwSV, vom 18. April 2017)

WGK 2: Hazard to waters

  - Chemical safety assessment

A chemical safety assessment has not been performed for the preparation/for the substances indicated in section 3.

## SECTION 16. Other information

Text of hazard (H) indications mentioned in section 2-3 of the sheet:

| Flam. Liq. 2 | Flammable liquid, category 2 |
| --- | --- |
| Flam. Liq. 3 | Flammable liquid, category 3 |
| Repr. 2 | Reproductive toxicity, category 2 |
| Acute Tox. 3 | Acute toxicity, category 3 |
| STOT SE 1 | Specific target organ toxicity - single exposure, category 1 |
| Acute Tox. 4 | Acute toxicity, category 4 |
| Asp. Tox. 1 | Aspiration hazard, category 1 |
| STOT RE 2 | Specific target organ toxicity - repeated exposure, category 2 |
| Eye Dam. 1 | Serious eye damage, category 1 |
| Eye Irrit. 2 | Eye irritation, category 2 |
| Skin Irrit. 2 | Skin irritation, category 2 |
| Skin Sens. 1 | Skin sensitization, category 1 |
| Skin Sens. 1A | Skin sensitization, category 1A |
| Skin Sens. 1B | Skin sensitization, category 1B |

# BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO

Printed on 16/01/2025

29/31

Replaced revision:1 (Dated: 04/09/2023)

| STOT SE 3 | Specific target organ toxicity - single exposure, category 3 |
| --- | --- |
| STOT SE 2 | Specific target organ toxicity - single exposure, category 2 |
| Aquatic Acute 1 | Hazardous to the aquatic environment, acute toxicity, category 1 |
| Aquatic Chronic 1 | Hazardous to the aquatic environment, chronic toxicity, category 1 |
| Aquatic Chronic 2 | Hazardous to the aquatic environment, chronic toxicity, category 2 |
| Aquatic Chronic 3 | Hazardous to the aquatic environment, chronic toxicity, category 3 |
| H225 | Highly flammable liquid and vapour. |
| H226 | Flammable liquid and vapour. |
| H361d | Suspected of damaging the unborn child. |
| H301 | Toxic if swallowed. |
| H311 | Toxic in contact with skin. |
| H331 | Toxic if inhaled. |
| H370 | Causes damage to organs. |
| H302 | Harmful if swallowed. |
| H312 | Harmful in contact with skin. |
| H332 | Harmful if inhaled. |
| H304 | May be fatal if swallowed and enters airways. |
| H373 | May cause damage to organs through prolonged or repeated exposure. |
| H318 | Causes serious eye damage. |
| H319 | Causes serious eye irritation. |
| H315 | Causes skin irritation. |
| H317 | May cause an allergic skin reaction. |
| H336 | May cause drowsiness or dizziness. |
| H371 | May cause damage to organs. |
| H400 | Very toxic to aquatic life. |
| H410 | Very toxic to aquatic life with long lasting effects. |
| H411 | Toxic to aquatic life with long lasting effects. |
| H412 | Harmful to aquatic life with long lasting effects. |
| EUH066 | Repeated exposure may cause skin dryness or cracking. |

### LEGEND:

- ADR: European Agreement concerning the carriage of Dangerous goods by Road

- ATE: Acute Toxicity Estimate

- CAS: Chemical Abstract Service Number

- CE50: Effective concentration (required to induce a 50% effect)

- CE: Identifier in ESIS (European archive of existing substances)

- CLP: Regulation (EC) 1272/2008

- DNEL: Derived No Effect Level

- EmS: Emergency Schedule

- GHS: Globally Harmonized System of classification and labeling of chemicals

- IATA DGR: International Air Transport Association Dangerous Goods Regulation

- IC50: Immobilization Concentration 50%

- IMDG: International Maritime Code for dangerous goods

- IMO: International Maritime Organization

- INDEX: Identifier in Annex VI of CLP

- LC50: Lethal Concentration 50%

- LD50: Lethal dose 50%

- OEL: Occupational Exposure Level

- PBT: Persistent, bioaccumulative and toxic

- PEC: Predicted environmental Concentration

- PEL: Predicted exposure level

| Suarez Company | Revision nr. 2 Dated 16/01/2025 Printed on 16/01/2025 30/31 Replaced revision:1 (Dated: 04/09/2023) |
| --- | --- |
| BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO |  |
| - PMT: Persistent, mobile and toxic - PNEC: Predicted no effect concentration - REACH: Regulation (EC) 1907/2006 - RID: Regulation concerning the international transport of dangerous goods by train - TLV: Threshold Limit Value - TLV CEILING: Concentration that should not be exceeded during any time of occupational exposure. - TWA: Time-weighted average exposure limit - TWA STEL: Short-term exposure limit - VOC: Volatile organic Compounds - vPvB: Very persistent and very bioaccumulative - vPvM: Very persistent and very mobile - WGK: Water hazard classes (German). GENERAL BIBLIOGRAPHY - Regulation (EC) 1907/2006 (REACH) of the European Parliament - Regulation (EC) 1272/2008 (CLP) of the European Parliament - Regulation (EU) 2020/878 (II Annex of REACH Regulation) - Regulation (EC) 790/2009 (I Atp. CLP) of the European Parliament - Regulation (EU) 286/2011 (II Atp. CLP) of the European Parliament - Regulation (EU) 618/2012 (III Atp. CLP) of the European Parliament - Regulation (EU) 487/2013 (IV Atp. CLP) of the European Parliament - Regulation (EU) 944/2013 (V Atp. CLP) of the European Parliament - Regulation (EU) 605/2014 (VI Atp. CLP) of the European Parliament - Regulation (EU) 2015/1221 (VII Atp. CLP) of the European Parliament - Regulation (EU) 2016/918 (VIII Atp. CLP) of the European Parliament - Regulation (EU) 2016/1179 (IX Atp. CLP) - Regulation (EU) 2017/776 (X Atp. CLP) - Regulation (EU) 2018/669 (XI Atp. CLP) - Regulation (EU) 2019/521 (XII Atp. CLP) - Delegated Regulation (UE) 2018/1480 (XIII Atp. CLP) - Regulation (EU) 2019/1148 - Delegated Regulation (UE) 2020/217 (XIV Atp. CLP) - Delegated Regulation (UE) 2020/1182 (XV Atp. CLP) - Delegated Regulation (UE) 2021/643 (XVI Atp. CLP) - Delegated Regulation (UE) 2021/849 (XVII Atp. CLP) - Delegated Regulation (UE) 2022/692 (XVIII Atp. CLP) - Delegated Regulation (UE) 2023/707 - Delegated Regulation (UE) 2023/1434 (XIX Atp. CLP) - Delegated Regulation (UE) 2023/1435 (XX Atp. CLP) - Delegated Regulation (UE) 2024/197 (XXI Atp. CLP) - The Merck Index. - 10th Edition - Handling Chemical Safety - INRS - Fiche Toxicologique (toxicological sheet) - Patty - Industrial Hygiene and Toxicology - N.I. Sax - Dangerous properties of Industrial Materials-7, 1989 Edition - IFA GESTIS website - ECHA website - Database of SDS models for chemicals - Ministry of Health and ISS (Istituto Superiore di Sanità) - Italy Note for users: The information contained in the present sheet are based on our own knowledge on the date of the last version. Users must verify the suitability and thoroughness of provided information according to each specific use of the product. This document must not be regarded as a guarantee on any specific product property. The use of this product is not subject to our direct control; therefore, users must, under their own responsibility, comply with the current health and safety laws and regulations. The producer is relieved from any liability arising from improper uses. Provide appointed staff with adequate training on how to use chemical products. CALCULATION METHODS FOR CLASSIFICATION Chemical and physical hazards: Product classification derives from criteria established by the CLP Regulation, Annex I, Part 2. The data for evaluation of chemical-physical properties are reported in section 9. Health hazards: Product classification is based on calculation methods as per Annex I of CLP, Part 3, unless determined otherwise in Section 11. Environmental hazards: Product classification is based on calculation methods as per Annex I of CLP, Part 4, unless determined otherwise in Section 12. |  |

| Suarez Company | Revision nr. 2 Dated 16/01/2025 Printed on 16/01/2025 31/31 Replaced revision:1 (Dated: 04/09/2023) |
| --- | --- |
| BLK0035-2 - SWEET HOME - PROFUMATORE AMBIENTE MUSCHIO BIANCO |  |
| Changes to previous review: The following sections were modified: 03 / 04 / 08 / 09 / 10 / 11 / 12 / 13 / 14 / 15 / 16. |  |