| Suarez Company |  | Revision nr. 1 Dated 03/12/2024 First compilation Printed on 03/12/2024 1/35 |  |
| --- | --- | --- | --- |
| BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO |  |  |  |
| Safety Data Sheet According to Annex II to REACH - Regulation (EU) 2020/878 |  |  |  |
| SECTION 1. Identification of the substance/mixture and of the company/undertaking |  |  |  |
| - Product identifier Code: BLK0276-2 Product name SWEET HOME - PROFUMATORE AMBIENTE SANDALO UFI : U6KJ-F3SM-UX0Q-6532 - Relevant identified uses of the substance or mixture and uses advised against |  |  |  |
| Identified Uses | Industrial Professional | Consumer |  |
| Room deodorant | - - |  |  |
| Uses Advised Against |  |  |  |
| Do not use for uses other than those indicated |  |  |  |
| 1.3. Details of the supplier of the safety data sheet Name Full address District and Country | Suarez Company Via Pergolesi , 1 ang. Via Monteverdi 20811 Cesano Maderno Italia |  |  |
|  | Tel. +39 0362 659766 |  |  |
|  | www.suarezcompany.it |  |  |
| e-mail address of the competent person |  |  |  |
| responsible for the Safety Data Sheet | info@suarezcompany.it |  |  |
| 1.4. Emergency telephone number For urgent inquiries refer to | Suarez Company |  |  |
|  | +39 0362 659766 |  |  |
|  | (8:30 - 12:30 13:30-17:30) |  |  |
| SECTION 2. Hazards identification |  |  |  |
| 2.1. Classification of the substance or mixture The product is classified as hazardous pursuant to the provisions set forth in (EC) Regulation 1272/2008 (CLP) (and subsequent amendments and supplements). The product thus requires a safety datasheet that complies with the provisions of (EU) Regulation 2020/878. Any additional information concerning the risks for health and/or the environment are given in sections 11 and 12 of this sheet. Hazard classification and indication: Flammable liquid, category 2 H225 Highly flammable liquid and vapour. Eye irritation, category 2 H319 Causes serious eye irritation. Skin sensitization, category 1A H317 May cause an allergic skin reaction. Hazardous to the aquatic environment, chronic toxicity, H412 Harmful to aquatic life with long lasting effects. category 3 |  |  |  |

# BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO

Printed on 03/12/2024 2/35

### 2.2. Label elements

Hazard labelling pursuant to EC Regulation 1272/2008 (CLP) and subsequent amendments and supplements. Hazard pictograms:

| Signal words: | Danger |
| --- | --- |

| Hazard statements: |  |
| --- | --- |
| H225 | Highly flammable liquid and vapour. |

| H319 | Causes serious eye irritation. |
| --- | --- |

| H317 | May cause an allergic skin reaction. |
| --- | --- |

| H412 | Harmful to aquatic life with long lasting effects. |
| --- | --- |

| Precautionary statements: |  |
| --- | --- |
| P101 | If medical advice is needed, have product container or label at hand. |

| P102 | Keep out of reach of children. |
| --- | --- |

| P210 | Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. |
| --- | --- |

| P302+P352 | IF ON SKIN: Wash with plenty of water / . . . |
| --- | --- |

| P305+P351+P338 | IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. |
| --- | --- |

| P333+P313 | If skin irritation or rash occurs: Get medical advice / attention. |
| --- | --- |

| P337+P313 | If eye irritation persists: Get medical advice / attention. |
| --- | --- |

| P501 | Dispose of contents / container in accordance with current regulations. |
| --- | --- |

| Contains: | CINNAMALDEHYDE |
| --- | --- |
|  | (4-TERT-BUTYLCYCLOHEXYL) ACETATE |
|  | Reaction mass of 1-[(1R*,6S*)-2,2,6-trimethylcyclohexyl]hexan-3-ol and 1-[(1S*,6S*)-2,2,6-trimethylcyclohexyl]hexan-3-ol |
|  | CINEOLE |
|  | benzyl salicylate |
|  | linalool |

2.3. Other hazards

# BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO

Printed on 03/12/2024 3/35

On the basis of available data, the product does not contain any PBT or vPvB in percentage ≥ than 0,1%. The product does not contain substances with endocrine disrupting properties in concentration ≥ 0.1%.

## SECTION 3. Composition/information on ingredients

### Substances

Information not relevant

### Mixtures

Contains:

| Identification | x = Conc. % | Classification (EC) 1272/2008 (CLP) |
| --- | --- | --- |
| ETHANOL |  |  |
| INDEX 603-002-00-5 | 74 ≤ x < 78 | Flam. Liq. 2 H225, Eye Irrit. 2 H319 |
| EC 200-578-6 |  | Eye Irrit. 2 H319: ≥ 50% |
| CAS 64-17-5 |  |  |
| REACH Reg. 01-2119457610-43 |  |  |
| (4-TERT-BUTYLCYCLOHEXYL) ACETATE |  |  |
| INDEX - | 7 ≤ x < 8 | Skin Sens. 1B H317 |
| EC 250-954-9 |  |  |
| CAS 32210-23-4 |  |  |
| REACH Reg. 01-2119976286-24 |  |  |
| DIPROPYLENE GLYCOL MONOMETHYL ETHER |  |  |
| INDEX - | 3,5 ≤ x < 4 | Substance with a community workplace exposure limit. |
| EC 252-104-2 |  |  |
| CAS 34590-94-8 |  |  |
| REACH Reg. 01-2119450011-60 |  |  |
| 2-ETHYL-4-(2,2,3-TRIMETHYL-3-CYCLOPENTEN-1-YL)-2-BUTEN-1- OL |  |  |
| INDEX - | 0,7 ≤ x < 0,8 | Eye Irrit. 2 H319, Aquatic Acute 1 H400 M=1, Aquatic Chronic 1 H410 M=1 |
| EC 248-908-8 |  |  |
| CAS 28219-61-6 |  |  |
| REACH Reg. 01-2119529224-45 |  |  |
| REACTION MASS OF 2-METHYLBUTYL SALICYLATE AND PENTYL SALICYLATE |  |  |
| INDEX - | 0,35 ≤ x < 0,4 | Acute Tox. 4 H302, Aquatic Acute 1 H400 M=1, Aquatic Chronic 2 H411 |
| EC 911-280-7 |  | LD50 Oral: 2000 mg/kg |
| CAS - |  |  |
| REACH Reg. 01-2119969444-27 |  |  |
| linalool |  |  |
| INDEX 603-235-00-2 | 0,3 ≤ x < 0,35 | Eye Irrit. 2 H319, Skin Irrit. 2 H315, Skin Sens. 1B H317 |
| EC 201-134-4 |  |  |

# BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO

Printed on 03/12/2024 4/35

| CAS 78-70-6 |  |  |
| --- | --- | --- |
| REACH Reg. 01-2119474016-42 |  |  |
| benzyl salicylate |  |  |
| INDEX 607-754-00-5 | 0,2 ≤ x < 0,25 | Eye Irrit. 2 H319, Skin Sens. 1B H317, Aquatic Chronic 3 H412 |
| EC 204-262-9 |  |  |
| CAS 118-58-1 |  |  |
| REACH Reg. 01-2119969442-31 |  |  |
| 2,6-di-tert-butyl-p-cresol |  |  |
| INDEX - | 0,1 ≤ x < 0,15 | Aquatic Chronic 1 H410 M=1 |
| EC 204-881-4 |  |  |
| CAS 128-37-0 |  |  |
| REACH Reg. 01-2119565113-46 |  |  |

(±) trans—

| 3,3-dimethyl-5-(2,2,3-trimethyl- cyclopent-3-en-1-yl)-pent-4-en-2-ol |  |  |
| --- | --- | --- |
| INDEX 603-150-00-0 | 0,1 ≤ x < 0,15 | Skin Irrit. 2 H315, Aquatic Acute 1 H400 M=1, Aquatic Chronic 1 H410 M=1 |
| EC 411-580-3 |  |  |
| CAS 107898-54-4 |  |  |
| REACH Reg. 01-0000015895-58 |  |  |
| CINEOLE |  |  |
| INDEX - | 0,1 ≤ x < 0,15 | Flam. Liq. 3 H226, Skin Sens. 1B H317 |
| EC 207-431-5 |  |  |
| CAS 470-82-6 |  |  |
| REACH Reg. 01-2119967772-24 |  |  |
| Reaction mass of 1-[(1R*,6S*)-2,2,6-trimethylcyclohexyl]hexan-3-ol and 1-[(1S*,6S*)-2,2,6- trimethylcyclohexyl]hexan-3-ol |  |  |
| INDEX - | 0,1 ≤ x < 0,15 | Skin Sens. 1B H317, Aquatic Acute 1 H400 M=1, Aquatic Chronic 1 H410 M=1 |
| EC 947-716-8 |  |  |
| CAS - |  |  |
| REACH Reg. 01-2120768938-30 |  |  |
| CINNAMALDEHYDE |  |  |
| INDEX 606-155-00-6 | 0,05 ≤ x < 0,1 | Skin Sens. 1A H317 |
| EC 203-213-9 |  | Skin Sens. 1A H317: ≥ 0,01% |
| CAS 104-55-2 |  |  |
| REACH Reg. 01-2119935242-45 |  |  |
| ALPHA-CEDRENE |  |  |
| INDEX - | 0 < x < 0,025 | Asp. Tox. 1 H304, Aquatic Acute 1 H400 M=10, Aquatic Chronic 1 H410 M=10 |
| EC 207-418-4 |  |  |
| CAS 469-61-4 |  |  |
| acetic acid ...% |  |  |
| INDEX 607-002-00-6 | 0 < x < 0,05 | Flam. Liq. 3 H226, Skin Corr. 1A H314, Eye Dam. 1 H318, Classification note according to Annex VI to the CLP Regulation: B |
| EC 200-580-7 |  | Skin Corr. 1A H314: ≥ 90%, Skin Corr. 1B H314: ≥ 25% - < 90%, Skin Corr. 1C H314: ≥ 25% - < 90%, Skin Irrit. 2 H315: ≥ 1% - < 25%, Eye Dam. 1 H318: ≥ 25%, Eye Irrit. 2 H319: ≥ 10% - < 25% |
| CAS 64-19-7 |  |  |

# BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO

Printed on 03/12/2024 5/35

| REACH Reg. 01-2119475328-30 |  |  |
| --- | --- | --- |
| METHANOL |  |  |
| INDEX 603-001-00-X | 0 < x < 0,05 | Flam. Liq. 2 H225, Acute Tox. 3 H301, Acute Tox. 3 H311, Acute Tox. 3 H331, STOT SE 1 H370 |
| EC 200-659-6 |  | STOT SE 2 H371: ≥ 3% - < 10% |
| CAS 67-56-1 |  | ATE Oral: 100 mg/kg, ATE Dermal: 300 mg/kg, ATE Inhalation vapours: 3 mg/l |
| REACH Reg. 01-2119433307-44 |  |  |
| HEPTANE |  |  |
| INDEX 601-008-00-2 | 0 < x < 0,05 | Flam. Liq. 2 H225, Asp. Tox. 1 H304, Skin Irrit. 2 H315, STOT SE 3 H336, Aquatic Acute 1 H400 M=1, Aquatic Chronic 1 H410 M=1, Classification note according to Annex VI to the CLP Regulation: C |
| EC 205-563-8 |  |  |
| CAS 142-82-5 |  |  |
| REACH Reg. 01-2119457603-38 |  |  |
| ACETONE |  |  |
| INDEX 606-001-00-8 | 0 < x < 0,05 | Flam. Liq. 2 H225, Eye Irrit. 2 H319, STOT SE 3 H336, EUH066 |
| EC 200-662-2 |  |  |
| CAS 67-64-1 |  |  |
| REACH Reg. 01-2119471330-49 |  |  |
| toluene |  |  |
| INDEX 601-021-00-3 | 0 < x < 0,05 | Flam. Liq. 2 H225, Repr. 2 H361d, Asp. Tox. 1 H304, STOT RE 2 H373, Skin Irrit. 2 H315, STOT SE 3 H336 |
| EC 203-625-9 |  |  |
| CAS 108-88-3 |  |  |
| REACH Reg. 01-2119471310-51- |  |  |
| phenol |  |  |
| INDEX 604-001-00-2 | 0 < x < 0,05 | Muta. 2 H341, Acute Tox. 3 H301, Acute Tox. 3 H311, Acute Tox. 3 H331, STOT RE 2 H373, Skin Corr. 1B H314, Eye Dam. 1 H318 |
| EC 203-632-7 |  | Skin Corr. 1B H314: ≥ 3%, Skin Irrit. 2 H315: ≥ 1% - < 3% |
| CAS 108-95-2 |  | LD50 Oral: 282 mg/kg bw, LD50 Dermal: 525 mg/kg bw, ATE Inhalation mists/powders: 0,501 mg/l |
| REACH Reg. 01-2119471329-32 |  |  |

The full wording of hazard (H) phrases is given in section 16 of the sheet.

## SECTION 4. First aid measures

### 4.1. Description of first aid measures

In case of doubt or in the presence of symptoms contact a doctor and show him this document. In case of more severe symptoms, ask for immediate medical aid.

EYES: Remove, if present, contact lenses if the situation allows you to do so easily. Wash immediately with plenty of water for at least 15 minutes, opening the eyelids fully. Get medical advice/attention.

SKIN: Take off immediately all contaminated clothing. Wash immediately and thoroughly with running water (and soap if possible). Get medical advice/attention. Avoid further contact with contaminated clothing.

INGESTION: Do not induce vomiting unless explicitly authorised by a doctor. Do not give anything by mouth to an unconscious person. Get medical advice/attention.

INHALATION: Remove victim to fresh air, away from the accident scene. Get medical advice/attention. Rescuer protection

It is good practice for rescuers lending support to a person who has been exposed to a chemical substance or to a mixture to wear personal protective equipment. The nature of such protection depends on the hazard level of the substance or mixture, on the type of exposure and on the extent of the

| Suarez Company | Revision nr. 1 Dated 03/12/2024 First compilation Printed on 03/12/2024 6/35 |
| --- | --- |
| BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO |  |
| contamination. In the absence of other more specific indications, use of disposable gloves in the event of possible contact with body fluids is recommended. For the type of PPE suitable for the characteristics of the substance or mixture, see section 8. 4.2. Most important symptoms and effects, both acute and delayed Specific information on symptoms and effects caused by the product are unknown. DELAYED EFFECTS: Based on the information currently available, there are no known cases of delayed effects following exposure to this product. 4.3. Indication of any immediate medical attention and special treatment needed If symptoms occur, whether acute or delayed, consult a doctor. Means to have available in the workplace for specific and immediate treatment Running water for skin and eye wash. SECTION 5. Firefighting measures - Extinguishing media SUITABLE EXTINGUISHING EQUIPMENT Extinguishing substances are: carbon dioxide, foam, chemical powder. For product loss or leakage that has not caught fire, water spray can be used to disperse flammable vapours and protect those trying to stem the leak. UNSUITABLE EXTINGUISHING EQUIPMENT Do not use jets of water. Water is not effective for putting out fires but can be used to cool containers exposed to flames to prevent explosions. - Special hazards arising from the substance or mixture HAZARDS CAUSED BY EXPOSURE IN THE EVENT OF FIRE Excess pressure may form in containers exposed to fire at a risk of explosion. Do not breathe combustion products. - Advice for firefighters GENERAL INFORMATION Use jets of water to cool the containers to prevent product decomposition and the development of substances potentially hazardous for health. Always wear full fire prevention gear. Collect extinguishing water to prevent it from draining into the sewer system. Dispose of contaminated water used for extinction and the remains of the fire according to applicable regulations. SPECIAL PROTECTIVE EQUIPMENT FOR FIRE-FIGHTERS Normal fire fighting clothing i.e. fire kit (BS EN 469), gloves (BS EN 659) and boots (HO specification A29 and A30) in combination with self-contained open circuit positive pressure compressed air breathing apparatus (BS EN 137). SECTION 6. Accidental release measures - Personal precautions, protective equipment and emergency procedures Block the leakage if there is no hazard. Wear suitable protective equipment (including personal protective equipment referred to under Section 8 of the safety data sheet) to prevent any contamination of skin, eyes and personal clothing. These indications apply for both processing staff and those involved in emergency procedures. Send away individuals who are not suitably equipped. Use explosion-proof equipment. Eliminate all sources of ignition (cigarettes, flames, sparks, etc.) from the leakage site. - Environmental precautions |  |

| Suarez Company | Revision nr. 1 Dated 03/12/2024 First compilation Printed on 03/12/2024 7/35 |
| --- | --- |
| BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO |  |
| The product must not penetrate into the sewer system or come into contact with surface water or ground water. - Methods and material for containment and cleaning up Collect the leaked product into a suitable container. Evaluate the compatibility of the container to be used, by checking section 10. Absorb the remainder with inert absorbent material. Make sure the leakage site is well aired. Contaminated material should be disposed of in compliance with the provisions set forth in point 13. - Reference to other sections Any information on personal protection and disposal is given in sections 8 and 13. SECTION 7. Handling and storage - Precautions for safe handling Keep away from heat, sparks and naked flames; do not smoke or use matches or lighters. Without adequate ventilation, vapours may accumulate at ground level and, if ignited, catch fire even at a distance, with the danger of backfire. Avoid bunching of electrostatic charges. When performing transfer operations involving large containers, connect to an earthing system and wear antistatic footwear. Vigorous stirring and flow through the tubes and equipment may cause the formation and accumulation of electrostatic charges. In order to avoid the risk of fires and explosions, never use compressed air when handling. Open containers with caution as they may be pressurised. Do not eat, drink or smoke during use. Avoid leakage of the product into the environment. - Conditions for safe storage, including any incompatibilities Store only in the original container. Store the containers sealed, in a well ventilated place, away from direct sunlight. Store in a cool and well ventilated place, keep far away from sources of heat, naked flames and sparks and other sources of ignition. Keep containers away from any incompatible materials, see section 10 for details. Storage class TRGS 510 (Germany): 3 - Specific end use(s) Information not available SECTION 8. Exposure controls/personal protection 8.1. Control parameters Regulatory references: BGR България НАРЕДБА № 13 ОТ 30 ДЕКЕМВРИ 2003 Г. ЗА ЗАЩИТА НА РАБОТЕЩИТЕ ОТ РИСКОВЕ, СВЪРЗАНИ С ЕКСПОЗИЦИЯ НА ХИМИЧНИ АГЕНТИ ПРИ РАБОТА (изм. ДВ. бр.5 от 17 Януари 2020г.) CZE Česká Republika NAŘÍZENÍ VLÁDY ze dne 10. května 2021, kterým se mění nařízení vlády č. 361/2007 Sb., kterým se stanoví podmínky ochrany zdraví při práci DEU Deutschland Forschungsgemeinschaft MAK- und BAT-Werte-Liste 2022 Ständige Senatskommission zur Prüfung gesundheitsschädlicher Arbeitsstoffe Mitteilung 58 DNK Danmark Bekendtgørelse om grænseværdier for stoffer og materialer - BEK nr 1458 af 13/12/2019 ESP España Límites de exposición profesional para agentes químicos en España 2023 FRA France Valeurs limites d'exposition professionnelle aux agents chimiques en FranceDécret n° 2021-1849 du 28 décembre 2021 FIN Suomi HTP-VÄRDEN 2020. Koncentrationer som befunnits skadliga. SOCIAL - OCH HÄLSOVÅRDSMINISTERIETS PUBLIKATIONER 2020:25 GRC Ελλάδα Π.Δ. 26/2020 (ΦΕΚ 50/Α` 6.3.2020) Εναρμόνιση της ελληνικής νομοθεσίας προς τις διατάξεις των οδηγιών 2017/2398/EΕ, 2019/130/ΕΕ και 2019/983/ΕΕ «για την τροποποίηση της οδηγίας 2004/37/ΕΚ ``σχετικά με |  |

| Suarez Company |  |  |  |  |  |  | Revision nr. 1 Dated 03/12/2024 First compilation Printed on 03/12/2024 8/35 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO |  |  |  |  |  |  |  |  |
|  |  |  | την προστασία των εργαζομένων από τους κινδύνους που συνδέονται με την έκθεση σε καρκινογόνους ή |  |  |  |  |  |
|  |  |  | μεταλλαξιγόνους παράγοντες κατά την εργασία``» |  |  |  |  |  |
| HUN | Magyarország |  | Az innovációért és technológiáért felelős miniszter 5/2020. (II. 6.) ITM rendelete a kémiai kóroki tényezők |  |  |  |  |  |
|  |  |  | hatásának kitett munkavállalók egészségének és biztonságának védelméről |  |  |  |  |  |
| HRV | Hrvatska |  | Pravilnik o izmjenama i dopunama Pravilnika o zaštiti radnika od izloženosti opasnimkemikalijama na radu, |  |  |  |  |  |
|  |  |  | graničnim vrijednostima izloženosti i biološkim graničnim vrijednostima (NN 1/2021) |  |  |  |  |  |
| ITA | Italia |  | Decreto Legislativo 9 Aprile 2008, n.81 |  |  |  |  |  |
| NOR | Norge |  | Forskrift om endring i forskrift om tiltaksverdier og grenseverdier for fysiske og kjemiske faktorer i |  |  |  |  |  |
|  |  |  | arbeidsmiljøet samt smitterisikogrupper for biologiske faktorer (forskrift om tiltaks- og grenseverdier), 21. |  |  |  |  |  |
|  |  |  | august 2018 nr. 1255 |  |  |  |  |  |
| NLD | Nederland |  | Arbeidsomstandighedenregeling. Lijst van wettelijke grenswaarden op grond van de artikelen 4.3, eerste |  |  |  |  |  |
|  |  |  | lid, en 4.16, eerste lid, van het Arbeidsomstandighedenbesluit |  |  |  |  |  |
| PRT | Portugal |  | Decreto-Lei n.º 1/2021 de 6 de janeiro, valores-limite de exposição profissional indicativos para os agentes |  |  |  |  |  |
|  |  |  | químicos. Decreto-Lei n.º 35/2020 de 13 de julho, proteção dos trabalhadores contra os riscos ligados à |  |  |  |  |  |
|  |  |  | exposição durante o trabalho a agentes cancerígenos ou mutagénicos |  |  |  |  |  |
| POL | Polska |  | Rozporządzenie ministra rozwoju, pracy i technologii z dnia 18 lutego 2021 r. Zmieniające rozporządzenie |  |  |  |  |  |
|  |  |  | w sprawie najwyższych dopuszczalnych stężeń i natężeń czynników szkodliwych dla zdrowia w |  |  |  |  |  |
|  |  |  | środowisku pracy |  |  |  |  |  |
| ROU | România |  | Hotărârea nr. 53/2021 pentru modificarea hotărârii guvernului nr. 1.218/2006, precum și pentru modificarea |  |  |  |  |  |
|  |  |  | și completarea hotărârii guvernului nr. 1.093/2006 |  |  |  |  |  |
| SWE | Sverige |  | Hygieniska gränsvärden, Arbetsmiljöverkets föreskrifter och allmänna råd om hygieniska gränsvärden (AFS |  |  |  |  |  |
|  |  |  | 2018:1) |  |  |  |  |  |
| SVK | Slovensko |  | NARIADENIE VLÁDY Slovenskej republiky z 12. augusta 2020, ktorým sa mení a dopĺňa nariadenie vlády |  |  |  |  |  |
|  |  |  | Slovenskej republiky č. 356/2006 Z. z. o ochrane zdravia zamestnancov pred rizikami súvisiacimi s |  |  |  |  |  |
|  |  |  | expozíciou karcinogénnym a mutagénnym faktorom pri práci v znení neskorších predpisov |  |  |  |  |  |
| SVN | Slovenija |  | Pravilnik o varovanju delavcev pred tveganji zaradi izpostavljenosti kemičnim snovem pri delu (Uradni list |  |  |  |  |  |
|  |  |  | RS, št. 100/01, 39/05, 53/07, 102/10, 43/11 – |  |  |  |  |  |
|  |  |  | ZVZD-1, 38/15, 78/18 in 78/19) |  |  |  |  |  |
| GBR | United Kingdom |  | EH40/2005 Workplace exposure limits (Fourth Edition 2020) |  |  |  |  |  |
| EU | OEL EU |  | Directive (EU) 2022/431; Directive (EU) 2019/1831; Directive (EU) 2019/130; Directive (EU) 2019/983; |  |  |  |  |  |
|  |  |  | Directive (EU) 2017/2398; Directive (EU) 2017/164; Directive 2009/161/EU; Directive 2006/15/EC; Directive |  |  |  |  |  |
|  |  |  | 2004/37/EC; Directive 2000/39/EC; Directive 98/24/EC; Directive 91/322/EEC. |  |  |  |  |  |
|  | TLV-ACGIH |  | ACGIH 2023 |  |  |  |  |  |
| ETHANOL |  |  |  |  |  |  |  |  |
| Threshold Limit Value |  |  |  |  |  |  |  |  |
| Type | Country | TWA/8h |  | STEL/15min |  | Remarks / Observations |  |  |
|  |  | mg/m3 | ppm | mg/m3 | ppm |  |  |  |
| TLV | BGR | 1000 |  |  |  |  |  |  |
| TLV | CZE | 1000 | 3000 |  |  |  |  |  |
| AGW | DEU | 960 | 500 | 1920 | 1000 |  |  |  |
| MAK | DEU | 960 | 500 | 1920 | 1000 |  |  |  |
| TLV | DNK | 1900 | 1000 |  |  |  |  |  |
| VLA | ESP |  |  | 1910 | 1000 |  |  |  |
| VLEP | FRA | 1900 | 1000 | 9500 | 5000 |  |  |  |
| HTP | FIN | 1900 | 1000 | 2500 | 1300 |  |  |  |
| TLV | GRC | 1900 | 1000 |  |  |  |  |  |
| AK | HUN | 1900 | 7600 |  |  |  |  |  |
| GVI/KGVI | HRV | 1900 | 1000 |  |  |  |  |  |
| TLV | NOR | 950 | 500 |  |  |  |  |  |
| TGG | NLD | 260 | 1900 |  |  |  |  |  |
| NDS/NDSCh | POL | 1900 |  |  |  |  |  |  |
| NGV/KGV | SWE | 1000 | 500 | 1900 | 1000 |  |  |  |
| NPEL | SVK | 960 | 500 | 1920 |  |  |  |  |
| WEL | GBR | 1920 | 1000 |  |  |  |  |  |
| TLV-ACGIH |  |  |  | 1884 | 1000 |  |  |  |
| Predicted no-effect concentration - PNEC |  |  |  |  |  |  |  |  |

| Suarez Company |  |  |  |  |  |  |  | Revision nr. 1 Dated 03/12/2024 First compilation Printed on 03/12/2024 9/35 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO |  |  |  |  |  |  |  |  |  |  |
| Normal value in fresh water |  |  |  |  | 0,96 | mg/l |  |  |  |  |
| Normal value in marine water |  |  |  |  | 0,79 | mg/l |  |  |  |  |
| Normal value for fresh water sediment |  |  |  |  | 3,6 | mg/kg |  |  |  |  |
| Normal value for marine water sediment |  |  |  |  | 2,9 | mg/kg |  |  |  |  |
| Normal value for water, intermittent release |  |  |  |  | 2,75 | mg/l |  |  |  |  |
| Normal value of STP microorganisms |  |  |  |  | 580 | mg/l |  |  |  |  |
| Normal value for the food chain (secondary poisoning) |  |  |  |  | 0,38 | mg/kg |  |  |  |  |
| Normal value for the terrestrial compartment |  |  |  |  | 0,63 | mg/kg |  |  |  |  |
| Health - Derived no-effect level - DNEL / DMEL |  |  |  |  |  |  |  |  |  |  |
|  |  | Effects on consumers |  |  |  | Effects on workers |  |  |  |  |
| Route of exposure |  | Acute local | Acute systemic | Chronic local | Chronic systemic | Acute local | Acute systemic | Chronic local | Chronic systemic |  |
| Oral |  |  |  |  | 87 mg/kg bw/d |  |  |  |  |  |
| Inhalation |  |  |  |  | 114 mg/m3 |  |  |  | 950 mg/m3 |  |
| Skin |  |  |  |  | 206 mg/kg bw/d |  |  |  | 343 mg/kg bw/d |  |
| (4-TERT-BUTYLCYCLOHEXYL) ACETATE |  |  |  |  |  |  |  |  |  |  |
| Predicted no-effect concentration - PNEC |  |  |  |  |  |  |  |  |  |  |
| Normal value in fresh water |  |  |  |  | 0,0053 | mg/l |  |  |  |  |
| Normal value in marine water |  |  |  |  | 0,00053 | mg/l |  |  |  |  |
| Normal value for fresh water sediment |  |  |  |  | 2,01 | mg/kg |  |  |  |  |
| Normal value for marine water sediment |  |  |  |  | 0,21 | mg/kg |  |  |  |  |
| Normal value for water, intermittent release |  |  |  |  | 0,053 | mg/l |  |  |  |  |
| Normal value of STP microorganisms |  |  |  |  | 12,2 | mg/l |  |  |  |  |
| Normal value for the food chain (secondary poisoning) |  |  |  |  | 66,67 | mg/kg |  |  |  |  |
| Normal value for the terrestrial compartment |  |  |  |  | 0,42 | mg/kg |  |  |  |  |
| DIPROPYLENE GLYCOL MONOMETHYL ETHER |  |  |  |  |  |  |  |  |  |  |
| Threshold Limit Value |  |  |  |  |  |  |  |  |  |  |
| Type | Country | TWA/8h |  |  | STEL/15min |  | Remarks / Observations |  |  |  |
|  |  | mg/m3 |  | ppm | mg/m3 | ppm |  |  |  |  |
| TLV | BGR | 308 |  |  |  |  | SKIN |  |  |  |
| TLV | CZE | 270 |  |  | 550 |  | SKIN |  |  |  |
| AGW | DEU | 310 |  | 50 | 310 | 50 |  |  |  |  |
| MAK | DEU | 310 |  | 50 | 310 | 50 |  |  |  |  |
| TLV | DNK | 300 |  | 50 |  |  |  |  |  |  |
| VLA | ESP | 308 |  | 50 |  |  | SKIN |  |  |  |
| VLEP | FRA | 308 |  | 50 |  |  | SKIN |  |  |  |
| HTP | FIN | 310 |  | 50 |  |  |  |  |  |  |
| TLV | GRC | 600 |  | 100 | 900 | 150 |  |  |  |  |
| AK | HUN | 308 |  |  | 308 |  |  |  |  |  |
| VLEP | ITA | 308 |  | 50 |  |  | SKIN |  |  |  |
| TLV | NOR | 300 |  | 50 |  |  | SKIN |  |  |  |
| VLE | PRT | 308 |  | 50 |  |  | SKIN |  |  |  |

| Suarez Company |  |  |  |  |  |  |  | Revision nr. 1 Dated 03/12/2024 First compilation Printed on 03/12/2024 10/35 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO |  |  |  |  |  |  |  |  |  |  |
| NDS/NDSCh | POL | 240 |  |  | 480 |  |  |  |  |  |
| NGV/KGV | SWE | 300 |  | 50 | 450 | 75 | SKIN |  |  |  |
| NPEL | SVK | 308 |  | 50 |  |  | SKIN |  |  |  |
| MV | SVN | 308 |  | 50 |  |  | SKIN |  |  |  |
| WEL | GBR | 308 |  | 50 |  |  | SKIN |  |  |  |
| OEL | EU | 308 |  | 50 |  |  | SKIN |  |  |  |
| TLV-ACGIH |  | 606 |  | 100 | 909 | 150 | SKIN |  |  |  |
| Predicted no-effect concentration - PNEC |  |  |  |  |  |  |  |  |  |  |
| Normal value in fresh water |  |  |  |  | 19 | mg/l |  |  |  |  |
| Normal value in marine water |  |  |  |  | 1,9 | mg/l |  |  |  |  |
| Normal value for fresh water sediment |  |  |  |  | 70,2 | mg/kg |  |  |  |  |
| Normal value for marine water sediment |  |  |  |  | 7,02 | mg/kg |  |  |  |  |
| Normal value for water, intermittent release |  |  |  |  | 190 | mg/l |  |  |  |  |
| Normal value of STP microorganisms |  |  |  |  | 4168 | mg/l |  |  |  |  |
| Normal value for the terrestrial compartment |  |  |  |  | 2,74 | mg/kg |  |  |  |  |
| Health - Derived no-effect level - DNEL / DMEL |  |  |  |  |  |  |  |  |  |  |
|  | Effects on consumers |  |  |  |  | Effects on workers |  |  |  |  |
| Route of exposure | Acute local |  | Acute systemic | Chronic local | Chronic systemic | Acute local | Acute systemic | Chronic local | Chronic systemic |  |
| Oral |  |  |  |  | 36 mg/kg bw/d |  |  |  |  |  |
| Inhalation |  |  |  |  | 37,2 mg/m3 |  |  |  | 308 mg/m3 |  |
| Skin |  |  |  |  | 121 mg/kg bw/d |  |  |  | 283 mg/kg bw/d |  |
| REACTION MASS OF 2-METHYLBUTYL SALICYLATE AND PENTYL SALICYLATE |  |  |  |  |  |  |  |  |  |  |
| Predicted no-effect concentration - PNEC |  |  |  |  |  |  |  |  |  |  |
| Normal value in fresh water |  |  |  |  | 0,00077 | mg/l |  |  |  |  |
| Normal value in marine water |  |  |  |  | 0,000077 | mg/l |  |  |  |  |
| Normal value for fresh water sediment |  |  |  |  | 0,389 | mg/kg |  |  |  |  |
| Normal value for marine water sediment |  |  |  |  | 0,039 | mg/kg |  |  |  |  |
| Normal value for water, intermittent release |  |  |  |  | 0,0077 | mg/l |  |  |  |  |
| Normal value of STP microorganisms |  |  |  |  | 10 | mg/l |  |  |  |  |
| Normal value for the food chain (secondary poisoning) |  |  |  |  | 80 | mg/kg |  |  |  |  |
| Normal value for the terrestrial compartment |  |  |  |  | 1,786 | mg/kg |  |  |  |  |
| Health - Derived no-effect level - DNEL / DMEL |  |  |  |  |  |  |  |  |  |  |
|  | Effects on consumers |  |  |  |  | Effects on workers |  |  |  |  |
| Route of exposure | Acute local |  | Acute systemic | Chronic local | Chronic systemic | Acute local | Acute systemic | Chronic local | Chronic systemic |  |
| Oral |  |  |  |  | 0,45 mg/kg bw/d |  |  |  |  |  |
| Inhalation |  |  |  |  | 0,78 mg/m3 |  |  |  | 3,17 mg/m3 |  |
| Skin |  |  |  |  | 0,45 mg/kg bw/d |  |  |  | 0,9 mg/kg bw/d |  |
| linalool |  |  |  |  |  |  |  |  |  |  |
| Predicted no-effect concentration - PNEC |  |  |  |  |  |  |  |  |  |  |
| Normal value in fresh water |  |  |  |  | 0,2 | mg/l |  |  |  |  |
| Normal value in marine water |  |  |  |  | 0,02 | mg/l |  |  |  |  |

| Suarez Company |  |  |  |  |  |  | Revision nr. 1 Dated 03/12/2024 First compilation Printed on 03/12/2024 11/35 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO |  |  |  |  |  |  |  |  |  |  |
| Normal value for fresh water sediment |  |  |  | 2,22 | mg/kg |  |  |  |  |  |
| Normal value for marine water sediment |  |  |  | 0,222 | mg/kg |  |  |  |  |  |
| Normal value for water, intermittent release |  |  |  | 2 | mg/l |  |  |  |  |  |
| Normal value of STP microorganisms |  |  |  | 10 | mg/l |  |  |  |  |  |
| Normal value for the food chain (secondary poisoning) |  |  |  | 7,8 | mg/kg food |  |  |  |  |  |
| Normal value for the terrestrial compartment |  |  |  | 0,327 | mg/kg |  |  |  |  |  |
| Health - Derived no-effect level - DNEL / DMEL |  |  |  |  |  |  |  |  |  |  |
|  | Effects on consumers |  |  |  | Effects on workers |  |  |  |  |  |
| Route of exposure | Acute local | Acute systemic | Chronic local | Chronic systemic | Acute local | Acute systemic |  | Chronic local | Chronic systemic |  |
| Oral |  | 1,2 mg/kg bw/d |  | 0,2 mg/kg bw/d |  |  |  |  |  |  |
| Inhalation |  | 4,1 mg/m3 |  | 0,7 mg/m3 |  | 16,5 mg/m3 |  |  | 2,8 mg/m3 |  |
| Skin | 1,5 mg/cm2 | 2,5 mg/kg bw/d | 1,5 mg/cm2 | 1,25 mg/kg bw/d | 3 mg/cm2 | 5 mg/kg bw/d |  | 3 mg/cm² | 2,5 mg/kg bw/d |  |
| benzyl salicylate |  |  |  |  |  |  |  |  |  |  |
| Predicted no-effect concentration - PNEC |  |  |  |  |  |  |  |  |  |  |
| Normal value in fresh water |  |  |  | 0,001 | mg/l |  |  |  |  |  |
| Normal value in marine water |  |  |  | 0 | mg/l |  |  |  |  |  |
| Normal value for fresh water sediment |  |  |  | 0,583 | mg/kg |  |  |  |  |  |
| Normal value for marine water sediment |  |  |  | 0,058 | mg/kg |  |  |  |  |  |
| Normal value for water, intermittent release |  |  |  | 0,01 | mg/l |  |  |  |  |  |
| Normal value of STP microorganisms |  |  |  | 10 | mg/l |  |  |  |  |  |
| Normal value for the food chain (secondary poisoning) |  |  |  | 80 | mg/kg |  |  |  |  |  |
| Normal value for the terrestrial compartment |  |  |  | 1,41 | mg/kg |  |  |  |  |  |
| Health - Derived no-effect level - DNEL / DMEL |  |  |  |  |  |  |  |  |  |  |
|  | Effects on consumers |  |  |  | Effects on workers |  |  |  |  |  |
| Route of exposure | Acute local | Acute systemic | Chronic local | Chronic systemic | Acute local | Acute systemic |  | Chronic local | Chronic systemic |  |
| Oral |  |  |  | 0,45 mg/kg bw/d |  |  |  |  |  |  |
| Inhalation |  |  |  | 0,78 mg/m3 |  |  |  |  | 3,17 mg/m3 |  |
| Skin (±) trans— |  |  |  | 0,45 mg/kg bw/d |  |  |  |  | 0,9 mg/kg bw/d |  |
| 3,3-dimethyl-5-(2,2,3-trimethyl-cyclopent-3-en-1-yl)-pent-4-en-2-ol |  |  |  |  |  |  |  |  |  |  |
| Predicted no-effect concentration - PNEC |  |  |  |  |  |  |  |  |  |  |
| Normal value in fresh water |  |  |  | 0,0012 | mg/l |  |  |  |  |  |
| Normal value in marine water |  |  |  | 0,00012 | mg/l |  |  |  |  |  |
| Normal value for fresh water sediment |  |  |  | 0,246 | mg/kg/d |  |  |  |  |  |
| Normal value for marine water sediment |  |  |  | 0,025 | mg/kg/d |  |  |  |  |  |
| Normal value for water, intermittent release |  |  |  | 0,012 | mg/l |  |  |  |  |  |
| Normal value of STP microorganisms |  |  |  | 0,1 | mg/l |  |  |  |  |  |
| Normal value for the food chain (secondary poisoning) |  |  |  | 222,2 | mg/kg food |  |  |  |  |  |
| Normal value for the terrestrial compartment |  |  |  | 0,048 | mg/kg |  |  |  |  |  |
| Reaction mass of 1-[(1R*,6S*)-2,2,6-trimethylcyclohexyl]hexan-3-ol and 1-[(1S*,6S*)-2,2,6-trimethylcyclohexyl]hexan-3-ol |  |  |  |  |  |  |  |  |  |  |

| Suarez Company |  |  |  |  |  |  | Revision nr. 1 Dated 03/12/2024 First compilation Printed on 03/12/2024 12/35 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO |  |  |  |  |  |  |  |  |  |
| Predicted no-effect concentration - PNEC |  |  |  |  |  |  |  |  |  |
| Normal value in fresh water |  |  |  | 0,001 | mg/l |  |  |  |  |
| Normal value in marine water |  |  |  | 0 | mg/l |  |  |  |  |
| Normal value for fresh water sediment |  |  |  | 0,5 | mg/kg/d |  |  |  |  |
| Normal value for marine water sediment |  |  |  | 0,05 | mg/kg/d |  |  |  |  |
| Normal value of STP microorganisms |  |  |  | 100 | mg/l |  |  |  |  |
| Normal value for the terrestrial compartment |  |  |  | 0,396 | mg/kg/d |  |  |  |  |
| Health - Derived no-effect level - DNEL / DMEL |  |  |  |  |  |  |  |  |  |
|  | Effects on consumers |  |  |  | Effects on workers |  |  |  |  |
| Route of exposure | Acute local | Acute systemic | Chronic local | Chronic systemic | Acute local | Acute systemic | Chronic local | Chronic systemic |  |
| Oral |  |  |  | 0,5 mg/kg bw/d |  |  |  |  |  |
| Inhalation |  |  |  | 1,74 mg/m3 |  |  |  | 9.87 mg/m3 |  |
| Skin |  |  |  | 1 mg/kg bw/d |  |  |  | 2,8 mg/kg bw/d |  |
| CINEOLE |  |  |  |  |  |  |  |  |  |
| Predicted no-effect concentration - PNEC |  |  |  |  |  |  |  |  |  |
| Normal value in fresh water |  |  |  | 0,057 | mg/l |  |  |  |  |
| Normal value in marine water |  |  |  | 0,0057 | mg/l |  |  |  |  |
| Normal value for fresh water sediment |  |  |  | 1,425 | mg/kg |  |  |  |  |
| Normal value for marine water sediment |  |  |  | 0,142 | mg/kg |  |  |  |  |
| Normal value for water, intermittent release |  |  |  | 0,57 | mg/l |  |  |  |  |
| Normal value of STP microorganisms |  |  |  | 10 | mg/l |  |  |  |  |
| Normal value for the food chain (secondary poisoning) |  |  |  | 40 | mg/kg |  |  |  |  |
| Normal value for the terrestrial compartment |  |  |  | 0,25 | mg/kg |  |  |  |  |
| Health - Derived no-effect level - DNEL / DMEL |  |  |  |  |  |  |  |  |  |
|  | Effects on consumers |  |  |  | Effects on workers |  |  |  |  |
| Route of exposure | Acute local | Acute systemic | Chronic local | Chronic systemic | Acute local | Acute systemic | Chronic local | Chronic systemic |  |
| Oral |  |  |  | 600 mg/kg bw/d |  |  |  |  |  |
| Inhalation |  |  |  | 1,74 mg/m3 |  |  |  | 7,05 mg/m3 |  |
| Skin |  |  |  | 1 mg/kg bw/d |  |  |  | 2 mg/kg bw/d |  |
| 2,6-di-tert-butyl-p-cresol |  |  |  |  |  |  |  |  |  |
| Predicted no-effect concentration - PNEC |  |  |  |  |  |  |  |  |  |
| Normal value in fresh water |  |  |  | 0,000199 | mg/l |  |  |  |  |
| Normal value in marine water |  |  |  | 0,02 | µg/l |  |  |  |  |
| Normal value for fresh water sediment |  |  |  | 0,0996 | mg/kg |  |  |  |  |
| Normal value for marine water sediment |  |  |  | 0,00996 | mg/kg |  |  |  |  |
| Normal value for water, intermittent release |  |  |  | 0,00199 | mg/l |  |  |  |  |
| Normal value of STP microorganisms |  |  |  | 0,17 | mg/l |  |  |  |  |
| Normal value for the food chain (secondary poisoning) |  |  |  | 8,33 | mg/kg |  |  |  |  |
| Normal value for the terrestrial compartment |  |  |  | 0,04769 | mg/kg |  |  |  |  |
| Health - Derived no-effect level - DNEL / DMEL |  |  |  |  |  |  |  |  |  |
|  | Effects on consumers |  |  |  | Effects on workers |  |  |  |  |
| Route of exposure | Acute local | Acute systemic | Chronic local | Chronic | Acute local | Acute | Chronic local | Chronic |  |

| Suarez Company |  |  |  |  |  |  |  | Revision nr. 1 Dated 03/12/2024 First compilation Printed on 03/12/2024 13/35 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | systemic |  | systemic |  | systemic |  |
| Oral |  |  |  |  | 0,25 mg/kg bw/d |  |  |  |  |  |
| Inhalation |  |  |  |  | 0,86 mg/m3 |  |  |  | 3,5 mg/m3 |  |
| Skin |  |  |  |  | 0,25 mg/kg bw/d |  |  |  | 0,5 mg/kg bw/d |  |
| CINNAMALDEHYDE |  |  |  |  |  |  |  |  |  |  |
| Predicted no-effect concentration - PNEC |  |  |  |  |  |  |  |  |  |  |
| Normal value in fresh water |  |  |  |  | 0,021 | mg/l |  |  |  |  |
| Normal value in marine water |  |  |  |  | 0,002 | mg/l |  |  |  |  |
| Normal value for fresh water sediment |  |  |  |  | 0,021 | mg/kg |  |  |  |  |
| Normal value for marine water sedimen |  |  |  |  | 0,002 | mg/kg |  |  |  |  |
| Normal value for water, intermittent release |  |  |  |  | 0,21 | mg/l |  |  |  |  |
| Normal value of STP microorganisms |  |  |  |  | 7,1 | mg/l |  |  |  |  |
| Normal value for the terrestrial compartment |  |  |  |  | 0,004 | mg/kg |  |  |  |  |
| Health - Derived no-effect level - DNEL / DMEL |  |  |  |  |  |  |  |  |  |  |
|  | Effects on consumers |  |  |  |  | Effects on workers |  |  |  |  |
| Route of exposure | Acute local |  | Acute systemic | Chronic local | Chronic systemic | Acute local | Acute systemic | Chronic local | Chronic systemic |  |
| Oral |  |  |  |  | 1,37 mg/kg bw/d |  |  |  |  |  |
| Inhalation |  |  |  |  | 2,4 mg/m3 |  |  |  | 13,6 mg/m3 |  |
| Skin |  |  |  |  | 1,37 mg/kg bw/d |  |  |  | 3,85 mg/kg bw/d |  |
| METHANOL |  |  |  |  |  |  |  |  |  |  |
| Threshold Limit Value |  |  |  |  |  |  |  |  |  |  |
| Type | Country | TWA/8h |  |  | STEL/15min |  | Remarks / Observations |  |  |  |
|  |  | mg/m3 |  | ppm | mg/m3 | ppm |  |  |  |  |
| TLV | BGR | 50 |  |  |  |  | SKIN |  |  |  |
| TLV | CZE | 250 |  |  | 1000 |  | SKIN |  |  |  |
| AGW | DEU | 270 |  | 200 | 1080 | 800 | SKIN |  |  |  |
| MAK | DEU | 270 |  | 200 | 1080 | 800 | SKIN |  |  |  |
| TLV | DNK | 260 |  | 200 |  |  |  |  |  |  |
| VLA | ESP | 266 |  | 200 |  |  | SKIN |  |  |  |
| VLEP | FRA | 260 |  | 200 | 1300 | 1000 | SKIN |  |  |  |
| HTP | FIN | 270 |  | 200 | 330 | 250 | SKIN |  |  |  |
| TLV | GRC | 260 |  | 200 | 325 | 250 |  |  |  |  |
| AK | HUN | 260 |  |  | 1040 |  |  |  |  |  |
| GVI/KGVI | HRV | 260 |  | 200 |  |  | SKIN |  |  |  |
| VLEP | ITA | 260 |  | 200 |  |  | SKIN |  |  |  |
| TLV | NOR | 130 |  | 100 |  |  | SKIN |  |  |  |
| TGG | NLD | 133 |  | 100 |  |  | SKIN |  |  |  |
| VLE | PRT | 260 |  | 200 |  |  | SKIN |  |  |  |
| NDS/NDSCh | POL | 100 |  |  | 300 |  |  |  |  |  |
| NGV/KGV | SWE | 250 |  | 200 | 350 | 250 | SKIN |  |  |  |
| NPEL | SVK | 260 |  | 200 |  |  | SKIN |  |  |  |

t

| Suarez Company |  |  |  |  |  |  |  | Revision nr. 1 Dated 03/12/2024 First compilation Printed on 03/12/2024 14/35 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO |  |  |  |  |  |  |  |  |  |  |  |
| WEL | GBR | 266 |  | 200 | 333 | 250 | SKIN |  |  |  |  |
| OEL | EU | 260 |  | 200 |  |  | SKIN |  |  |  |  |
| TLV-ACGIH |  | 262 |  | 200 | 328 | 250 |  |  |  |  |  |
| Predicted no-effect concentration - PNEC |  |  |  |  |  |  |  |  |  |  |  |
| Normal value in fresh water |  |  |  |  | 20,8 | mg/l |  |  |  |  |  |
| Normal value in marine water |  |  |  |  | 2,08 | mg/l |  |  |  |  |  |
| Normal value for fresh water sediment |  |  |  |  | 77 | mg/kg |  |  |  |  |  |
| Normal value for marine water sediment |  |  |  |  | 7,7 | mg/kg |  |  |  |  |  |
| Normal value for water, intermittent release |  |  |  |  | 1540 | mg/l |  |  |  |  |  |
| Normal value of STP microorganisms |  |  |  |  | 100 | mg/l |  |  |  |  |  |
| Normal value for the terrestrial compartment |  |  |  |  | 100 | mg/kg |  |  |  |  |  |
| Health - Derived no-effect level - DNEL / DMEL |  |  |  |  |  |  |  |  |  |  |  |
|  | Effects on consumers |  |  |  |  | Effects on workers |  |  |  |  |  |
| Route of exposure | Acute local |  | Acute systemic | Chronic local | Chronic systemic | Acute local | Acute systemic |  | Chronic local | Chronic systemic |  |
| Oral |  |  | 4 mg/kg bw/d |  | 4 mg/kg bw/d |  |  |  |  |  |  |
| Inhalation | 26 mg/m3 |  | 26 mg/m3 | 26 mg/m3 | 26 mg/m3 | 130 mg/m3 | 130 mg/m3 |  | 130 mg/m3 | 130 mg/m3 |  |
| Skin |  |  | 4 mg/kg bw/d |  | 4 mg/kg bw/d |  | 20 mg/kg bw/d |  |  | 20 mg/kg bw/d |  |
| HEPTANE |  |  |  |  |  |  |  |  |  |  |  |
| Threshold Limit Value |  |  |  |  |  |  |  |  |  |  |  |
| Type | Country | TWA/8h |  |  | STEL/15min |  | Remarks / Observations |  |  |  |  |
|  |  | mg/m3 |  | ppm | mg/m3 | ppm |  |  |  |  |  |
| TLV | BGR | 1600 |  |  |  |  |  |  |  |  |  |
| TLV | CZE | 1000 |  | 240 | 2000 | 480 |  |  |  |  |  |
| MAK | DEU | 2100 |  | 500 | 2100 | 500 |  |  |  |  |  |
| TLV | DNK | 820 |  | 200 |  |  |  |  | E |  |  |
| VLA | ESP | 2085 |  | 500 |  |  |  |  | Como n-Eptano |  |  |
| VLEP | FRA | 1668 |  | 400 | 2085 | 500 |  |  |  |  |  |
| TLV | GRC | 2000 |  | 500 | 2000 | 500 |  |  |  |  |  |
| AK | HUN | 2000 |  |  |  |  |  |  |  |  |  |
| GVI/KGVI | HRV | 2085 |  | 500 |  |  | SKIN |  |  |  |  |
| VLEP | ITA | 2085 |  | 500 |  |  |  |  |  |  |  |
| TLV | NOR | 800 |  | 200 |  |  |  |  |  |  |  |
| TGG | NLD | 1200 |  |  | 1600 |  |  |  |  |  |  |
| VLE | PRT | 2085 |  | 500 |  |  |  |  |  |  |  |
| NDS/NDSCh | POL | 1200 |  |  | 2000 |  |  |  |  |  |  |
| TLV | ROU | 2085 |  | 500 |  |  |  |  |  |  |  |
| NGV/KGV | SWE | 800 |  | 200 | 1200 (C) | 300 (C) |  |  |  |  |  |
| NPEL | SVK | 2085 |  | 500 |  |  |  |  |  |  |  |
| MV | SVN | 2085 |  | 500 | 2085 | 500 |  |  |  |  |  |
| WEL | GBR | 2085 |  | 500 |  |  |  |  |  |  |  |
| OEL | EU | 2085 |  | 500 |  |  |  |  |  |  |  |
| TLV-ACGIH |  | 1639 |  | 400 | 2049 | 500 |  |  |  |  |  |

| Suarez Company |  |  |  |  |  |  | Revision nr. 1 Dated 03/12/2024 First compilation Printed on 03/12/2024 15/35 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO |  |  |  |  |  |  |  |  |
| ACETONE |  |  |  |  |  |  |  |  |
| Threshold Limit Value |  |  |  |  |  |  |  |  |
| Type | Country | TWA/8h |  | STEL/15min |  | Remarks / Observations |  |  |
|  |  | mg/m3 | ppm | mg/m3 | ppm |  |  |  |
| TLV | BGR | 600 |  | 1400 |  |  |  |  |
| TLV | CZE | 800 | 331,2 | 1500 | 621 |  |  |  |
| AGW | DEU | 1200 | 500 | 2400 (C) | 1000 (C) |  |  |  |
| MAK | DEU | 1200 | 500 | 2400 | 1000 |  |  |  |
| TLV | DNK | 600 | 250 |  |  |  | E |  |
| VLEP | FRA | 1210 | 500 | 2420 | 1000 |  |  |  |
| HTP | FIN | 1200 | 500 | 1500 | 630 |  |  |  |
| TLV | GRC | 1780 |  | 3560 |  |  |  |  |
| AK | HUN | 1210 |  |  |  |  |  |  |
| GVI/KGVI | HRV | 1210 | 500 |  |  |  |  |  |
| VLEP | ITA | 1210 | 500 |  |  |  |  |  |
| TLV | NOR | 295 | 125 |  |  |  |  |  |
| TGG | NLD | 1210 |  | 2420 |  |  |  |  |
| VLE | PRT | 1210 | 500 |  |  |  |  |  |
| NDS/NDSCh | POL | 600 |  | 1800 |  |  |  |  |
| TLV | ROU | 1210 | 500 |  |  |  |  |  |
| NGV/KGV | SWE | 600 | 250 | 1200 (C) | 500 (C) |  |  |  |
| NPEL | SVK | 1210 | 500 |  |  |  |  |  |
| MV | SVN | 1210 | 500 | 2420 | 1000 |  |  |  |
| WEL | GBR | 1210 | 500 | 3620 | 1500 |  |  |  |
| OEL | EU | 1210 | 500 |  |  |  |  |  |
| TLV-ACGIH |  |  | 250 |  | 500 |  |  |  |
| toluene |  |  |  |  |  |  |  |  |
| Threshold Limit Value |  |  |  |  |  |  |  |  |
| Type | Country | TWA/8h |  | STEL/15min |  | Remarks / Observations |  |  |
|  |  | mg/m3 | ppm | mg/m3 | ppm |  |  |  |
| TLV | BGR | 150 |  | 300 |  |  |  |  |
| TLV | CZE | 200 |  | 500 |  | SKIN |  |  |
| AGW | DEU | 190 | 50 | 760 | 200 | SKIN |  |  |
| MAK | DEU | 190 | 50 | 760 | 200 |  |  |  |
| TLV | DNK | 94 | 25 |  |  | SKIN |  |  |
| VLA | ESP | 192 | 50 | 384 | 100 | SKIN |  |  |
| VLEP | FRA | 76,8 | 20 | 384 | 100 | SKIN |  |  |
| HTP | FIN | 81 | 25 | 380 | 100 | SKIN |  |  |
| TLV | GRC | 192 | 50 | 384 | 100 |  |  |  |
| AK | HUN | 190 |  | 760 |  |  |  |  |
| GVI/KGVI | HRV | 192 | 50 | 384 | 100 | SKIN |  |  |
| VLEP | ITA | 192 | 50 |  |  | SKIN |  |  |

| Suarez Company |  |  |  |  |  |  |  | Revision nr. 1 Dated 03/12/2024 First compilation Printed on 03/12/2024 16/35 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO |  |  |  |  |  |  |  |  |  |  |  |
| TLV | NOR | 94 |  | 25 |  |  | SKIN |  |  |  |  |
| TGG | NLD | 150 |  |  | 384 |  |  |  |  |  |  |
| VLE | PRT | 192 |  | 50 | 384 | 100 | SKIN |  |  |  |  |
| NDS/NDSCh | POL | 100 |  |  | 200 |  |  |  |  |  |  |
| TLV | ROU | 192 |  | 50 | 384 | 100 | SKIN |  |  |  |  |
| NGV/KGV | SWE | 192 |  | 50 | 384 | 100 | SKIN |  |  |  |  |
| NPEL | SVK | 192 |  | 50 | 384 |  | SKIN |  |  |  |  |
| MV | SVN | 192 |  | 50 | 384 | 100 | SKIN |  |  |  |  |
| WEL | GBR | 191 |  | 50 | 384 | 100 | SKIN |  |  |  |  |
| OEL | EU | 192 |  | 50 | 384 | 100 | SKIN |  |  |  |  |
| TLV-ACGIH |  | 75,4 |  | 20 |  |  |  |  |  |  |  |
| Predicted no-effect concentration - PNEC |  |  |  |  |  |  |  |  |  |  |  |
| Normal value in fresh water |  |  |  |  | 0,68 | mg/l |  |  |  |  |  |
| Normal value in marine water |  |  |  |  | 0,68 | mg/l |  |  |  |  |  |
| Normal value for fresh water sediment |  |  |  |  | 16,39 | mg/kg/d |  |  |  |  |  |
| Normal value for marine water sediment |  |  |  |  | 16,39 | mg/kg/d |  |  |  |  |  |
| Normal value for water, intermittent release |  |  |  |  | 0,68 | mg/l |  |  |  |  |  |
| Normal value of STP microorganisms |  |  |  |  | 13,61 | mg/l |  |  |  |  |  |
| Normal value for the food chain (secondary poisoning) |  |  |  |  | 2,89 | mg/kg soil dw |  |  |  |  |  |
| Health - Derived no-effect level - DNEL / DMEL |  |  |  |  |  |  |  |  |  |  |  |
|  | Effects on consumers |  |  |  |  | Effects on workers |  |  |  |  |  |
| Route of exposure | Acute local |  | Acute systemic | Chronic local | Chronic systemic | Acute local | Acute systemic |  | Chronic local | Chronic systemic |  |
| Oral |  |  |  |  | 8,13 mg/kg bw/d |  |  |  |  |  |  |
| Inhalation | 226 mg/m3 |  | 226 mg/m3 | 56,5 mg/m3 | 56,5 mg/m3 | 384 mg/m3 | 384 mg/m3 |  | 192 mg/m3 | 192 mg/m3 |  |
| Skin |  |  |  |  | 226 mg/kg bw/d |  |  |  |  | 384 mg/kg bw/d |  |
| phenol |  |  |  |  |  |  |  |  |  |  |  |
| Threshold Limit Value |  |  |  |  |  |  |  |  |  |  |  |
| Type | Country | TWA/8h |  |  | STEL/15min |  | Remarks / Observations |  |  |  |  |
|  |  | mg/m3 |  | ppm | mg/m3 | ppm |  |  |  |  |  |
| TLV | BGR | 7,8 |  |  |  |  | SKIN |  |  |  |  |
| TLV | CZE | 7,5 |  |  | 15 |  | SKIN |  |  |  |  |
| AGW | DEU | 8 |  | 2 | 16 | 4 | SKIN |  |  |  |  |
| TLV | DNK | 4 |  | 1 |  |  |  |  |  |  |  |
| VLA | ESP | 8 |  | 2 | 16 | 4 | SKIN |  |  |  |  |
| VLEP | FRA | 7,8 |  | 2 | 15,6 | 4 | SKIN |  |  |  |  |
| HTP | FIN | 8 |  | 2 | 16 | 4 | SKIN |  |  |  |  |
| TLV | GRC | 8 |  | 2 | 16 | 4 |  |  |  |  |  |
| AK | HUN | 7,8 |  |  |  |  |  |  |  |  |  |
| GVI/KGVI | HRV | 7,8 |  | 2 |  |  | SKIN |  |  |  |  |
| VLEP | ITA | 8 |  | 2 | 16 | 4 | SKIN |  |  |  |  |
| TLV | NOR | 4 |  | 1 |  |  | SKIN |  |  |  |  |
| TGG | NLD | 8 |  |  |  |  | SKIN |  |  |  |  |

| Suarez Company |  |  |  |  |  |  |  | Revision nr. 1 Dated 03/12/2024 First compilation Printed on 03/12/2024 17/35 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO |  |  |  |  |  |  |  |  |  |  |
| VLE | PRT | 8 |  | 2 | 16 | 4 | SKIN |  |  |  |
| NDS/NDSCh | POL | 7,8 |  |  | 16 |  |  |  |  |  |
| TLV | ROU | 8 |  | 2 | 16 | 4 | SKIN |  |  |  |
| NGV/KGV | SWE | 4 |  | 1 | 8 | 2 | SKIN |  |  |  |
| NPEL | SVK | 7,8 |  | 2 |  |  |  |  |  |  |
| MV | SVN | 8 |  | 2 | 16 | 4 | SKIN |  |  |  |
| WEL | GBR | 7,8 |  | 2 | 16 | 4 | SKIN |  |  |  |
| OEL | EU | 8 |  | 2 | 16 | 4 | SKIN |  |  |  |
| TLV-ACGIH |  | 19,2 |  | 5 |  |  |  |  |  |  |
| Predicted no-effect concentration - PNEC |  |  |  |  |  |  |  |  |  |  |
| Normal value in fresh water |  |  |  |  | 0,008 | mg/l |  |  |  |  |
| Normal value in marine water |  |  |  |  | 0,001 | mg/l |  |  |  |  |
| Normal value for fresh water sediment |  |  |  |  | 0,091 | mg/kg/d |  |  |  |  |
| Normal value for marine water sediment |  |  |  |  | 0,009 | mg/kg/d |  |  |  |  |
| Normal value for water, intermittent release |  |  |  |  | 0,031 | mg/l |  |  |  |  |
| Normal value of STP microorganisms |  |  |  |  | 2,1 | mg/l |  |  |  |  |
| Normal value for the terrestrial compartment |  |  |  |  | 0,136 | mg/kg/d |  |  |  |  |
| Health - Derived no-effect level - DNEL / DMEL |  |  |  |  |  |  |  |  |  |  |
|  | Effects on consumers |  |  |  |  | Effects on workers |  |  |  |  |
| Route of exposure | Acute local |  | Acute systemic | Chronic local | Chronic systemic | Acute local | Acute systemic | Chronic local | Chronic systemic |  |
| Oral |  |  |  |  | 0,4 mg/kg bw/d |  |  |  |  |  |
| Inhalation |  |  |  |  | 1,32 mg/m3 | 16 mg/m3 |  |  | 8 mg/m3 |  |
| Skin |  |  |  |  | 0,4 mg/kg bw/d |  |  |  | 1,23 mg/kg bw/d |  |
| acetic acid ...% |  |  |  |  |  |  |  |  |  |  |
| Threshold Limit Value |  |  |  |  |  |  |  |  |  |  |
| Type | Country | TWA/8h |  |  | STEL/15min |  | Remarks / Observations |  |  |  |
|  |  | mg/m3 |  | ppm | mg/m3 | ppm |  |  |  |  |
| TLV | BGR | 25 |  |  | 37 |  |  |  |  |  |
| TLV | CZE | 25 |  |  | 35 |  |  |  |  |  |
| AGW | DEU | 25 |  | 10 | 50 | 20 |  |  |  |  |
| MAK | DEU | 25 |  | 10 | 50 | 20 |  |  |  |  |
| TLV | DNK | 25 |  | 10 |  |  |  |  |  |  |
| VLA | ESP | 25 |  | 10 | 37 | 15 |  |  |  |  |
| VLEP | FRA |  |  |  | 25 | 10 |  |  |  |  |
| HTP | FIN | 13 |  | 5 | 25 | 10 |  |  |  |  |
| TLV | GRC | 25 |  | 10 | 37 | 15 |  |  |  |  |
| AK | HUN | 25 |  |  | 25 |  |  |  |  |  |
| GVI/KGVI | HRV | 25 |  | 10 |  |  |  |  |  |  |
| TLV | NOR | 25 |  | 10 |  |  |  |  |  |  |
| VLE | PRT | 25 |  | 10 |  |  |  |  |  |  |
| NDS/NDSCh | POL | 15 |  |  | 30 |  |  |  |  |  |
| NGV/KGV | SWE | 13 |  | 5 | 25 | 10 |  |  |  |  |

| BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO |  |  |  |  |  |  |  | Printed on 03/12/2024 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | 18/35 |  |  |
| NPEL | SVK | 25 | 10 |  |  |  |  |  |  |  |
| MV | SVN | 25 | 10 |  |  |  |  |  |  |  |
| OEL | EU | 25 | 10 | 50 | 20 |  |  |  |  |  |
| TLV-ACGIH |  | 25 | 10 | 37 | 15 |  |  |  |  |  |
| Predicted no-effect concentration - PNEC |  |  |  |  |  |  |  |  |  |  |
| Normal value in fresh water |  |  |  | 3,058 |  |  | mg/l |  |  |  |
| Normal value in marine water |  |  |  | 0,306 |  |  | mg/l |  |  |  |
| Normal value for fresh water sediment |  |  |  | 11,36 |  |  | mg/kg |  |  |  |
| Normal value for marine water sediment |  |  |  | 1,136 |  |  | mg/kg |  |  |  |
| Normal value for water, intermittent release |  |  |  | 30,58 |  |  | mg/l |  |  |  |
| Normal value of STP microorganisms |  |  |  | 85 |  |  | mg/l |  |  |  |
| Normal value for the terrestrial compartment |  |  |  | 0,47 |  |  | mg/kg |  |  |  |
| Health - Derived no-effect level - DNEL / DMEL Effects on |  |  |  |  |  | Effects on |  |  |  |  |
| consumers |  |  |  |  |  | workers |  |  |  |  |
| Route of exposure Acute local Acute systemic |  |  | Chronic local | Chronic systemic |  | Acute local | Acute Chronic local systemic |  | Chronic systemic |  |
| Inhalation 25 mg/m3 |  |  | 25 mg/m3 |  |  | 25 mg/m3 |  | 25 mg/m3 |  |  |
| Legend: (C) = CEILING ; INHAL = Inhalable Fraction ; RESP = Respirable Fraction ; THORA = Thoracic Fraction. VND = hazard identified but no DNEL/PNEC available ; NEA = no exposure expected ; NPI = no hazard identified ; LOW = low hazard ; MED = medium hazard ; HIGH = high hazard. 8.2. Exposure controls As the use of adequate technical equipment must always take priority over personal protective equipment, make sure that the workplace is well aired through effective local aspiration. When choosing personal protective equipment, ask your chemical substance supplier for advice. Personal protective equipment must be CE marked, showing that it complies with applicable standards. Provide an emergency shower with face and eye wash station. HAND PROTECTION Protect hands with category III work gloves. The following should be considered when choosing work glove material (see standard EN 374): compatibility, degradation, permeability time. The work gloves' resistance to chemical agents should be checked before use, as it can be unpredictable. The gloves' wear time depends on the duration and type of use. SKIN PROTECTION Wear category II professional long-sleeved overalls and safety footwear (see Regulation 2016/425 and standard EN ISO 20344). Wash body with soap and water after removing protective clothing. Consider the appropriateness of providing antistatic clothing in the case of working environments in which there is a risk of explosion. EYE PROTECTION Wear airtight protective goggles (see standard EN ISO 16321). |  |  |  |  |  |  |  |  |  |  |

# BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO

Printed on 03/12/2024 19/35

RESPIRATORY PROTECTION

Respiratory protection devices must be used if the technical measures adopted are not suitable for restricting the worker's exposure to the threshold values considered. Use a mask with a type A filter whose class (1, 2 or 3) must be chosen according to the limit of use concentration. (see standard EN 14387).

If the substance considered is odourless or its olfactory threshold is higher than the corresponding TLV-TWA and in the case of an emergency, wear open-circuit compressed air breathing apparatus (in compliance with standard EN 137) or external air-intake breathing apparatus (in compliance with standard EN 138). For a correct choice of respiratory protection device, see standard EN 529.

ENVIRONMENTAL EXPOSURE CONTROLS

The emissions generated by manufacturing processes, including those generated by ventilation equipment, should be checked to ensure compliance with environmental standards.

Product residues must not be indiscriminately disposed of with waste water or by dumping in waterways.

## SECTION 9. Physical and chemical properties

  - Information on basic physical and chemical properties

| Properties | Value | Information |
| --- | --- | --- |
| Appearance | liquid | Method:internal |
|  |  | Temperature: 20 °C |
| Colour | brown | Remark:Visual |
|  |  | Temperature: 20 °C |
| Odour | characteristic | Method:internal |
| Odour threshold | 18,8 | Method:internal |
|  |  | Remark:mg/m3 |
|  |  | Substance:ETHANOL |
|  |  | Temperature: 20 °C |
| Melting point / freezing point | -114 °C | Method:HSDB 2015; INRS 2011 |
|  |  | Substance:ETHANOL |
| Initial boiling point | 78,5 °C | Method:HSDB 2015; INRS 2011 |
|  |  | Substance:ETHANOL |
|  |  | Initial boiling point: 78 °C |
| Flammability | Easily flammable liquid and vapors. | Method:ISO 3679:2023 |
| Lower explosive limit | 3,3 % (v/v) | Method:internal |
|  |  | Remark:INAIL, 2018 |
|  |  | Substance:ETHANOL |
| Upper explosive limit | 19 % (v/v) | Method:internal |
|  |  | Remark:INAIL, 2018 |
|  |  | Substance:ETHANOL |
| Flash point | 18 °C | Method:ISO 3679:2023 |
| Auto-ignition temperature | 363 °C | Method:internal |
|  |  | Remark:1013 hPa |
|  |  | Substance:ETHANOL |
| Decomposition temperature | not available | Reason for missing data:It only applies to authoritative substances and mixtures, organic peroxides and other substances and mixtures that they can decompose |
| pH | not available | Reason for missing data:This property is not relevant to the safety and classification of this product. |
| Kinematic viscosity | not available | Reason for missing data:This property is not |

# BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO

Printed on 03/12/2024 20/35

|  |  | relevant to the safety and classification of this product. |
| --- | --- | --- |
| Solubility | soluble in water | Method:internal |
|  |  | Temperature: 20 °C |
| Partition coefficient: n-octanol/water | -0,35 | Method:internal |
|  |  | Substance:ETHANOL |
|  |  | Temperature: 20 °C |
| Vapour pressure | 5,9 kPa | Method:INRS, 2011 |
|  |  | Substance:ETHANOL |
|  |  | Vapour pressure: 45 mmHg |
|  |  | Temperature: 20 °C |
| Density and/or relative density | not available |  |
| Relative vapour density | 1,6 | Method:HSDB, 2015; INRS, 2011 |
|  |  | Remark:(aria=1) HSDB, 2015; INRS, 2011 |
|  |  | Substance:ETHANOL |

Particle characteristics Median equivalent diameter

| Remark: | It only applies to solids |
| --- | --- |

| Remark: | It only applies to solids |
| --- | --- |

| Remark: | It only applies to solids |
| --- | --- |

Size distribution Dustiness

| Remark: | It only applies to solids |
| --- | --- |

| Remark: | It only applies to solids |
| --- | --- |

Specific surface area Shape

  - Other information

    - Information with regard to physical hazard classes

Information not available

    - Other safety characteristics

| Explosive properties | not available | Reason for missing data:Absent chemical groups associated with explosive properties in accordance with the provisions of Annex I, Part 2, chap. 2.1.4.3 of Reg. (EC) 1272/2008 - CLP |
| --- | --- | --- |
| Oxidising properties | not available | Reason for missing data:Absent requirements related to the presence of atoms or chemical bonds associated with oxidizing properties in the molecules of the components according to Annex I, Part 2, 2.13.4 Reg. (CE) 1272/2008 |

## SECTION 10. Stability and reactivity

### 10.1. Reactivity

There are no particular risks of reaction with other substances in normal conditions of use.

| Suarez Company | Revision nr. 1 Dated 03/12/2024 First compilation Printed on 03/12/2024 21/35 |
| --- | --- |
| BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO |  |
| DIPROPYLENE GLYCOL MONOMETHYL ETHER May react with: oxidising substances.When heated to decomposition releases: harsh fumes,zinc alloys. ACETONE Decomposes under the effect of heat. toluene Avoid exposure to: light. - Chemical stability The product is stable in normal conditions of use and storage. - Possibility of hazardous reactions The vapours may also form explosive mixtures with the air. ETHANOL Risk of explosion on contact with: alkaline metals,alkaline oxides,calcium hypochlorite,sulphur monofluoride,acetic anhydride,acids,concentrated hydrogen peroxide,perchlorates,perchloric acid,perchloronitrile,mercury nitrate,nitric acid,silver,silver nitrate,ammonia,silver oxide,ammonia,strong oxidising agents,nitrogen dioxide.May react dangerously with: bromoacetylene,chlorine acetylene,bromine trifluoride,chromium trioxide,chromyl chloride,fluorine,potassium tert-butoxide,lithium hydride,phosphorus trioxide,black platinum,zirconium (IV) chloride,zirconium (IV) iodide.Forms explosive mixtures with: air. ACETONE Risk of explosion on contact with: bromine trifluoride,fluorine dioxide,hydrogen peroxide,nitrosyl chloride,2-methyl-1,3 butadiene,nitromethane,nitrosyl perchlorate.May react dangerously with: potassium tert-butoxide,alkaline hydroxides,bromine,bromoform,isoprene,sodium,sulphur dioxide,chromium trioxide,chromyl chloride,nitric acid,chloroform,peroxymonosulphuric acid,phosphoryl oxychloride,chromosulphuric acid,fluorine,strong oxidising agents,strong reducing agents.Develops flammable gas on contact with: nitrosyl perchlorate. toluene Risk of explosion on contact with: fuming sulphuric acid,nitric acid,silver perchlorate,nitrogen dioxide,non-metal halogenates,acetic acid,organic nitrocompounds.May form explosive mixtures with: air.May react dangerously with: strong oxidising agents,strong acids,sulphur. acetic acid ...% Risk of explosion on contact with: chromium (VI) oxide,potassium permanganate,sodium peroxide,perchloric acid,phosphorus chloride,hydrogen peroxide.May react dangerously with: alcohols,bromine pentafluoride,chlorosulphuric acid,dichromate-sulphuric acid,ethane diamine,ethylene glycol,potassiun hydroxide,strong bases,sodium hydroxide,strong oxidising agents,nitric acid,ammonium nitrate,potassium tert-butoxide,oleum.Forms explosive mixtures with: air. - Conditions to avoid Avoid overheating. Avoid bunching of electrostatic charges. Avoid all sources of ignition. ETHANOL Avoid exposure to: sources of heat,naked flames. |  |

| Suarez Company | Revision nr. 1 Dated 03/12/2024 First compilation Printed on 03/12/2024 22/35 |
| --- | --- |
| BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO |  |
| ACETONE Avoid exposure to: sources of heat,naked flames. acetic acid ...% Avoid exposure to: sources of heat,naked flames. - Incompatible materials ACETONE Incompatible with: acids,oxidising substances. acetic acid ...% Incompatible with: carbonates,hydroxides,phosphates,oxidising substances,bases. - Hazardous decomposition products In the event of thermal decomposition or fire, gases and vapours that are potentially dangerous to health may be released. ACETONE May develop: ketenes,irritant substances. SECTION 11. Toxicological information In the absence of experimental data for the product itself, health hazards are evaluated according to the properties of the substances it contains, using the criteria specified in the applicable regulation for classification. It is therefore necessary to take into account the concentration of the individual hazardous substances indicated in section 3, to evaluate the toxicological effects of exposure to the product. - Information on hazard classes as defined in Regulation (EC) No 1272/2008 Metabolism, toxicokinetics, mechanism of action and other information Information not available Information on likely routes of exposure METHANOL WORKERS: inhalation; contact with the skin. POPULATION: ingestion of contaminated food or water; contact with the skin of products containing the substance. toluene WORKERS: inhalation; contact with the skin. POPULATION: ingestion of contaminated food or water; inhalation of ambient air; contact with the skin of products containing the substance. Delayed and immediate effects as well as chronic effects from short and long-term exposure METHANOL The minimum lethal dose for humans by ingestion is considered to be in the range from 300 to 1000 mg/kg. Ingestion of 4-10 ml of the substance may cause permanent blindness in adult humans (IPCS). |  |

| Suarez Company | Revision nr. 1 Dated 03/12/2024 First compilation Printed on 03/12/2024 23/35 |
| --- | --- |
| BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO |  |
| toluene Toxic effect on the central and peripheral nervous system with encephalopathy and polyneuritis; irritating for the skin, conjunctiva, cornea and respiratory apparatus. Interactive effects toluene Certain drugs and other industrial products can interfere with the metabolism of the toluene. ACUTE TOXICITY ATE (Inhalation) of the mixture: Not classified (no significant component) ATE (Oral) of the mixture: Not classified (no significant component) ATE (Dermal) of the mixture: Not classified (no significant component) ETHANOL LD50 (Oral): > 5000 mg/kg Rat LC50 (Inhalation vapours): 120 mg/l/4h Pimephales promelas (4-TERT-BUTYLCYCLOHEXYL) ACETATE LD50 (Dermal): > 5000 mg/kg rabbit LD50 (Oral): 3,6 mg/kg rat DIPROPYLENE GLYCOL MONOMETHYL ETHER LD50 (Dermal): 10 mL/kg rabbit LD50 (Oral): > 5000 mg/kg rat LC50 (Inhalation vapours): 3,35 mg/L 7h REACTION MASS OF 2-METHYLBUTYL SALICYLATE AND PENTYL SALICYLATE LD50 (Dermal): > 2000 mg/kg rabbit LD50 (Oral): 2000 mg/kg rat linalool LD50 (Dermal): 5610 mg/kg rabbit LD50 (Oral): 2200 mg/kg mouse LC50 (Inhalation vapours): > 3,2 mg/l 90 min mouse benzyl salicylate LD50 (Dermal): > 2000 mg/kg bw rabbit LD50 (Oral): > 2000 mg/kg rat (±) trans— 3,3-dimethyl-5-(2,2,3-trimethyl-cyclopent-3-en-1-yl)-pent-4-en-2-ol LD50 (Dermal): > 2000 mg/kg rat LD50 (Oral): > 5000 mg/kg rat Reaction mass of 1-[(1R*,6S*)-2,2,6-trimethylcyclohexyl]hexan-3-ol and 1-[(1S*,6S*)-2,2,6-trimethylcyclohexyl]hexan-3-ol LD50 (Dermal): > 2000 mg/kg rat LD50 (Oral): > 18000 mg/kg rat CINEOLE LD50 (Dermal): > 2 g/kg rat LD50 (Oral): 4,3 g/kg rat 2,6-di-tert-butyl-p-cresol LD50 (Dermal): > 2000 mg/kg rat LD50 (Oral): 2930 mg/kg rat CINNAMALDEHYDE LD50 (Dermal): 620 mg/kg rabbit |  |

# BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO

Printed on 03/12/2024 24/35

| LD50 (Oral): | 1160 mg/kg guinea pig |
| --- | --- |
| LC50 (Inhalation vapours): | 68,889 mg/l/4h rat |

METHANOL

| LD50 (Oral): | > 1187 mg/kg Rat |
| --- | --- |
| LC50 (Inhalation vapours): | 43,68 mg/l/6h Rat |

toluene

| LD50 (Dermal): | > 5000 mg/kg bw Rabbit |
| --- | --- |
| LD50 (Oral): | > 5000 mg/kg bw Rat |
| LC50 (Inhalation vapours): | 100 ppm human |

phenol

| LD50 (Dermal): | 525 mg/kg bw Rat |
| --- | --- |
| LD50 (Oral): | 282 mg/kg bw Mouse |

acetic acid ...%

| LD50 (Dermal): | 1060 mg/kg Rabbit |
| --- | --- |
| LD50 (Oral): | 3310 mg/kg Rat |
| LC50 (Inhalation vapours): | 11,4 mg/l/4h Rat |

SKIN CORROSION / IRRITATION

Does not meet the classification criteria for this hazard class SERIOUS EYE DAMAGE / IRRITATION

Causes serious eye irritation RESPIRATORY OR SKIN SENSITISATION

Sensitising for the skin

GERM CELL MUTAGENICITY

Does not meet the classification criteria for this hazard class CARCINOGENICITY

Does not meet the classification criteria for this hazard class

toluene

Classified in Group 3 (not classifiable as a human carcinogen) by the International Agency for Research on Cancer (IARC) - (IARC, 1999). The US Environmental Protection Agency (EPA) affirms that "the data is inadequate for an assessment of the carcinogenic potential".

REPRODUCTIVE TOXICITY

Does not meet the classification criteria for this hazard class STOT - SINGLE EXPOSURE

Does not meet the classification criteria for this hazard class STOT - REPEATED EXPOSURE

Does not meet the classification criteria for this hazard class

# BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO

Printed on 03/12/2024 25/35

ASPIRATION HAZARD

Does not meet the classification criteria for this hazard class

### 11.2. Information on other hazards

Based on the available data, the product does not contain substances listed in the main European lists of potential or suspected endocrine disruptors with human health effects under evaluation.

## SECTION 12. Ecological information

This product is dangerous for the environment and the aquatic organisms. In the long term, it has negative effects on the aquatic environment.

### Toxicity

| Reaction mass of 1-[(1R*,6S*)-2,2,6-trimethylcyclohexyl]hexan-3-ol and 1-[(1S*,6S*)-2,2,6-trimethylcyclohexyl]hexan-3- ol |  |
| --- | --- |
| LC50 - for Fish | 999 mg/l/96h |
| EC50 - for Crustacea | 696 mg/l/48h |

| CINNAMALDEHYDE |  |
| --- | --- |
| LC50 - for Fish | > 3,5 mg/l/96h |
| EC50 - for Crustacea | 3,21 mg/l/48h |
| EC50 - for Algae / Aquatic Plants | 16,09 mg/l/72h |
| Chronic NOEC for Crustacea | 0,049 mg/l |

(±) trans—

| 3,3-dimethyl-5-(2,2,3-trimethyl-cyclopent-3- en-1-yl)-pent-4-en-2-ol |  |
| --- | --- |
| LC50 - for Fish | 1,2 mg/l/96h Oncorhynchus mykiss |
| EC50 - for Crustacea | 1 mg/l/48h |
| EC50 - for Algae / Aquatic Plants | 0,88 mg/l/72h |
| Chronic NOEC for Crustacea | 0,56 mg/l |
| Chronic NOEC for Algae / Aquatic Plants | 0,45 mg/l |

| toluene |  |
| --- | --- |
| LC50 - for Fish | 5,5 mg/l/96h Oncorhynchus kisutch |
| EC50 - for Crustacea | 3,78 mg/l |
| EC50 - for Algae / Aquatic Plants | 134 mg/l/3 h Chlorella vulgaris and Chlamydomonas angulosa |
| Chronic NOEC for Algae / Aquatic Plants | 10 mg/l/72 h Skeletonema costatum |

| phenol |  |
| --- | --- |
| LC50 - for Fish | 21,93 mg/l/14 d Poecilia reticulata |
| EC50 - for Crustacea | 3,1 mg/l/48h Ceriodaphnia dubia |
| EC50 - for Algae / Aquatic Plants | 61,1 mg/l/96h 96 h EC50 (cell number increase): 61.1 mg phenol/L (95 % confidence limits: 37.1 - 84.5 mg/L) |
| Chronic NOEC for Fish | 4 mg/l/14 d Poecilia reticulata |
| Chronic NOEC for Crustacea | 0,16 mg/l |

# BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO

Printed on 03/12/2024 26/35

| benzyl salicylate |  |
| --- | --- |
| LC50 - for Fish | 1,03 mg/l/96h Danio rerio |
| EC50 - for Crustacea | 1,16 mg/l/48h Daphnia magna |
| EC50 - for Algae / Aquatic Plants | 1,29 mg/l/72h Pseudokirchneriella subcapitata |
| Chronic NOEC for Crustacea | 0,894 mg/l Daphnia magna |
| Chronic NOEC for Algae / Aquatic Plants | 0,502 mg/l Pseudokirchneriella subcapitata |

| 2,6-di-tert-butyl-p-cresol |  |
| --- | --- |
| LC50 - for Fish | 0,199 mg/l/96h |
| EC50 - for Crustacea | 0,48 mg/l/48h |
| EC50 - for Algae / Aquatic Plants | > 0,4 mg/l/72h |
| Chronic NOEC for Fish | 0,053 mg/l |
| Chronic NOEC for Crustacea | 0,15 mg/l |

| HEPTANE |  |
| --- | --- |
| LC50 - for Fish | 375 mg/l/96h Tilapia mossambica |
| EC50 - for Crustacea | 82,5 mg/l/48h Daphnia magna |
| EC50 - for Algae / Aquatic Plants | 1,5 mg/l/72h Algae |

| (4-TERT-BUTYLCYCLOHEXYL) ACETATE |  |
| --- | --- |
| LC50 - for Fish | 8,6 mg/l/96h Cyprinus carpio |
| EC50 - for Crustacea | 5,3 mg/l/48h Daphnia magna |
| EC50 - for Algae / Aquatic Plants | 22 mg/l/72h Desmodesmus subspicatus |
| Chronic NOEC for Algae / Aquatic Plants | 6,8 mg/l Desmodesmus subspicatus |

| DIPROPYLENE GLYCOL MONOMETHYL ETHER |  |
| --- | --- |
| LC50 - for Fish | > 150 mg/l/96h |
| EC50 - for Crustacea | > 1000 mg/l/48h |
| EC50 - for Algae / Aquatic Plants | > 969 mg/l/72h |
| Chronic NOEC for Crustacea | 1000 mg/l 48h |
| Chronic NOEC for Algae / Aquatic Plants | 969 mg/l |

| CINEOLE |  |
| --- | --- |
| LC50 - for Fish | 57 mg/l/96h Oncorhynchus mykiss |
| EC50 - for Crustacea | > 100 mg/l/48h Daphnia magna |
| EC50 - for Algae / Aquatic Plants | 74 mg/l/72h |
| Chronic NOEC for Fish | 32 mg/l Oncorhynchus mykiss |
| Chronic NOEC for Crustacea | 100 mg/l Daphnia magna |
| Chronic NOEC for Algae / Aquatic Plants | 9,1 mg/l |

| ETHANOL |  |
| --- | --- |
| LC50 - for Fish | 14200 mg/l/96h |
| EC50 - for Crustacea | 454 mg/l/48h |

# BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO

Printed on 03/12/2024 27/35

| EC50 - for Algae / Aquatic Plants | 275 mg/l/72h |
| --- | --- |
| Chronic NOEC for Fish | 250 mg/l |
| Chronic NOEC for Crustacea | 96 mg/l |
| Chronic NOEC for Algae / Aquatic Plants | 11,5 mg/l |

| acetic acid ...% |  |
| --- | --- |
| LC50 - for Fish | > 1000 mg/l/96h |
| EC50 - for Crustacea | > 300,82 mg/l/48h |
| EC50 - for Algae / Aquatic Plants | > 1000 mg/l/72h |

| METHANOL |  |
| --- | --- |
| LC50 - for Fish | 15320 mg/l/96h Leuciscus idus melanotus |
| EC50 - for Crustacea | 18260 mg/l/96 h Daphnia magna |
| EC50 - for Algae / Aquatic Plants | 22000 mg/l/96 h Pseudokirchneriella subcapitata |
| Chronic NOEC for Fish | 3950 mg/l/96 h Danio rerio |
| Chronic NOEC for Crustacea | 7960 mg/l/96 h Mytilus edulis |

| linalool |  |
| --- | --- |
| LC50 - for Fish | 27,8 mg/l/96h Oncorhynchus mykiss |
| EC50 - for Crustacea | 59 mg/l/48h Daphnia magna |
| EC50 - for Algae / Aquatic Plants | 88,3 mg/l/72h Desmodesmus subspicatus |
| Chronic NOEC for Fish | < 3,5 mg/l Oncorhynchus mykiss |

| REACTION MASS OF 2-METHYLBUTYL SALICYLATE AND PENTYL SALICYLATE |  |
| --- | --- |
| LC50 - for Fish | 1,34 mg/l/96h |
| EC50 - for Crustacea | 0,88 mg/l/48h |
| EC50 - for Algae / Aquatic Plants | 0,49 mg/l/72h |
| Chronic NOEC for Algae / Aquatic Plants | 0,2 mg/l |

### Persistence and degradability

Reaction mass of 1-[(1R*,6S*)-2,2,6-trimethylcyclohexyl]hexan-3-ol and 1-[(1S*,6S*)-2,2,6-trimethylcyclohexyl]hexan-3-ol

NOT rapidly degradable

CINNAMALDEHYDE

Rapidly degradable (±) trans—

3,3-dimethyl-5-(2,2,3-trimethyl-cyclopent-3-en-1-yl)-pent-4-en-2-ol

Rapidly degradable

| toluene |  |
| --- | --- |
| Solubility in water | 100 - 1000 mg/l |

Rapidly degradable phenol

Rapidly degradable benzyl salicylate

Rapidly degradable

# BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO

Printed on 03/12/2024 28/35

| HEPTANE |  |
| --- | --- |
| Solubility in water | 0,1 - 100 mg/l |

2,6-di-tert-butyl-p-cresol Rapidly degradable

Rapidly degradable

(4-TERT-BUTYLCYCLOHEXYL) ACETATE

Rapidly degradable

| DIPROPYLENE GLYCOL MONOMETHYL ETHER |  |
| --- | --- |
| Solubility in water | 1000 - 10000 mg/l |

Rapidly degradable CINEOLE

Rapidly degradable

| ETHANOL |  |
| --- | --- |
| Solubility in water | 1000 - 10000 mg/l |

Rapidly degradable

| acetic acid ...% |  |
| --- | --- |
| Solubility in water | > 10000 mg/l |

Rapidly degradable

| METHANOL |  |
| --- | --- |
| Solubility in water | 1000 - 10000 mg/l |

Rapidly degradable ACETONE

Rapidly degradable linalool

Rapidly degradable

REACTION MASS OF 2-METHYLBUTYL SALICYLATE AND PENTYL SALICYLATE

Rapidly degradable

### Bioaccumulative potential

| CINNAMALDEHYDE |  |
| --- | --- |
| BCF | 147,91 |

(±) trans—

| 3,3-dimethyl-5-(2,2,3-trimethyl-cyclopent-3- en-1-yl)-pent-4-en-2-ol |  |
| --- | --- |
| BCF | 46 |

| toluene |  |
| --- | --- |
| Partition coefficient: n-octanol/water | 2,73 |
| BCF | 90 |

| phenol |  |
| --- | --- |
| Partition coefficient: n-octanol/water | 1,47 |
| BCF | 17,5 |

| 2,6-di-tert-butyl-p-cresol |  |
| --- | --- |
| Partition coefficient: n-octanol/water | 5,1 |
| BCF | 598,4 - |

# BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO

Printed on 03/12/2024 29/35

| HEPTANE |  |
| --- | --- |
| Partition coefficient: n-octanol/water | 4,5 |
| BCF | 552 |

| DIPROPYLENE GLYCOL MONOMETHYL ETHER |  |
| --- | --- |
| Partition coefficient: n-octanol/water | 0,0043 |

| CINEOLE |  |
| --- | --- |
| BCF | 155 l/kg |

| ETHANOL |  |
| --- | --- |
| Partition coefficient: n-octanol/water | -0,35 |

| acetic acid ...% |  |
| --- | --- |
| Partition coefficient: n-octanol/water | -0,17 |

| METHANOL |  |
| --- | --- |
| Partition coefficient: n-octanol/water | -0,77 |
| BCF | 0,2 |

| ACETONE |  |
| --- | --- |
| Partition coefficient: n-octanol/water | -0,23 |
| BCF | 3 |

| REACTION MASS OF 2-METHYLBUTYL SALICYLATE AND PENTYL SALICYLATE |  |
| --- | --- |
| BCF | 1170 |

### Mobility in soil

| CINNAMALDEHYDE |  |
| --- | --- |
| Partition coefficient: soil/water | 1,958 |

(±) trans—

| 3,3-dimethyl-5-(2,2,3-trimethyl-cyclopent-3- en-1-yl)-pent-4-en-2-ol |  |
| --- | --- |
| Partition coefficient: soil/water | 3,023 |

| toluene |  |
| --- | --- |
| Partition coefficient: soil/water | 2,31 |

| phenol |  |
| --- | --- |
| Partition coefficient: soil/water | 1,57 l/kg |

| 2,6-di-tert-butyl-p-cresol |  |
| --- | --- |
| Partition coefficient: soil/water | 4,2 |

# BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO

Printed on 03/12/2024 30/35

| HEPTANE |  |
| --- | --- |
| Partition coefficient: soil/water | 2,38 |

| CINEOLE |  |
| --- | --- |
| Partition coefficient: soil/water | 2,33 |

| acetic acid ...% |  |
| --- | --- |
| Partition coefficient: soil/water | 1,153 |

| REACTION MASS OF 2-METHYLBUTYL SALICYLATE AND PENTYL SALICYLATE |  |
| --- | --- |
| Partition coefficient: soil/water | 37 |

### Results of PBT and vPvB assessment

On the basis of available data, the product does not contain any PBT or vPvB in percentage ≥ than 0,1%.

### Endocrine disrupting properties

Based on the available data, the product does not contain substances listed in the main European lists of potential or suspected endocrine disruptors with environmental effects under evaluation.

### Other adverse effects

Information not available

## SECTION 13. Disposal considerations

### Waste treatment methods

Reuse, when possible. Product residues should be considered special hazardous waste. The hazard level of waste containing this product should be evaluated according to applicable regulations.

Disposal must be performed through an authorised waste management firm, in compliance with national and local regulations. Waste transportation may be subject to ADR restrictions.

The management of waste arising from the use or dispersal of this product must be organised in accordance with occupational safety regulations. See section 8 for possible need for PPE.

CONTAMINATED PACKAGING

Contaminated packaging must be recovered or disposed of in compliance with national waste management regulations.

## SECTION 14. Transport information

  - UN number or ID number

| ADR / RID, IMDG, IATA: | UN 1170 |
| --- | --- |

  - UN proper shipping name

| Suarez Company | Revision nr. 1 Dated 03/12/2024 First compilation Printed on 03/12/2024 31/35 |
| --- | --- |
| BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO |  |
| ADR / RID: ETHANOL (ETHYL ALCOHOL) or ETHANOL SOLUTION (ETHYL ALCOHOL SOLUTION) IMDG: ETHANOL (ETHYL ALCOHOL) or ETHANOL SOLUTION (ETHYL ALCOHOL SOLUTION) IATA: ETHANOL (ETHYL ALCOHOL) or ETHANOL SOLUTION (ETHYL ALCOHOL SOLUTION) - Transport hazard class(es) ADR / RID: Class: 3 Label: 3 IMDG: Class: 3 Label: 3 IATA: Class: 3 Label: 3 - Packing group ADR / RID, IMDG, IATA: II - Environmental hazards ADR / RID: NO IMDG: not marine pollutant IATA: NO |  |
| 14.6. Special precautions for user |  |
| ADR / RID: HIN - Kemler: 33 Limited Quantities: 1 L Special provision: 144, 601 | Tunnel restriction code: (D/E) |
| IMDG: EMS: F-E, S-D Limited Quantities: 1 L IATA: Cargo: Maximum quantity: 60 L Passengers: Maximum quantity: 5 L Special provision: A3, A58, A180 | Packaging instructions: 364 Packaging instructions: 353 |
| 14.7. Maritime transport in bulk according to IMO instruments |  |
| Information not relevant |  |
| SECTION 15. Regulatory information |  |
| 15.1. Safety, health and environmental regulations/legislation specific for the substance or mixture |  |
| Seveso Category - Directive 2012/18/EU: P5c |  |

# BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO

Printed on 03/12/2024 32/35

Restrictions relating to the product or contained substances pursuant to Annex XVII to EC Regulation 1907/2006

Product

| Point | 3 - 40 |
| --- | --- |

Contained substance

| Point | 75 |
| --- | --- |

Regulation (EU) 2019/1148 - on the marketing and use of explosives precursors

not applicable

Substances in Candidate List (Art. 59 REACH)

On the basis of available data, the product does not contain any SVHC in percentage ≥ than 0,1%. Substances subject to authorisation (Annex XIV REACH)

None

Substances subject to exportation reporting pursuant to Regulation (EU) 649/2012:

None

Substances subject to the Rotterdam Convention:

None

Substances subject to the Stockholm Convention:

None

Healthcare controls

Workers exposed to this chemical agent must not undergo health checks, provided that available risk-assessment data prove that the risks related to the workers' health and safety are modest and that the 98/24/EC directive is respected.

German regulation on the classification of substances hazardous to water (AwSV, vom 18. April 2017)

WGK 2: Hazard to waters

### 15.2. Chemical safety assessment

A chemical safety assessment has not been performed for the preparation/for the substances indicated in section 3.

## SECTION 16. Other information

Text of hazard (H) indications mentioned in section 2-3 of the sheet:

# BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO

Printed on 03/12/2024 33/35

| Flam. Liq. 2 | Flammable liquid, category 2 |
| --- | --- |
| Flam. Liq. 3 | Flammable liquid, category 3 |
| Muta. 2 | Germ cell mutagenicity, category 2 |
| Repr. 2 | Reproductive toxicity, category 2 |
| Acute Tox. 3 | Acute toxicity, category 3 |
| STOT SE 1 | Specific target organ toxicity - single exposure, category 1 |
| Acute Tox. 4 | Acute toxicity, category 4 |
| Asp. Tox. 1 | Aspiration hazard, category 1 |
| STOT RE 2 | Specific target organ toxicity - repeated exposure, category 2 |
| Skin Corr. 1A | Skin corrosion, category 1A |
| Skin Corr. 1B | Skin corrosion, category 1B |
| Skin Corr. 1C | Skin corrosion, category 1C |
| Skin Corr. 1 | Skin corrosion, category 1 |
| Eye Dam. 1 | Serious eye damage, category 1 |
| Eye Irrit. 2 | Eye irritation, category 2 |
| Skin Irrit. 2 | Skin irritation, category 2 |
| Skin Sens. 1A | Skin sensitization, category 1A |
| Skin Sens. 1B | Skin sensitization, category 1B |
| STOT SE 3 | Specific target organ toxicity - single exposure, category 3 |
| STOT SE 2 | Specific target organ toxicity - single exposure, category 2 |
| Aquatic Acute 1 | Hazardous to the aquatic environment, acute toxicity, category 1 |
| Aquatic Chronic 1 | Hazardous to the aquatic environment, chronic toxicity, category 1 |
| Aquatic Chronic 2 | Hazardous to the aquatic environment, chronic toxicity, category 2 |
| Aquatic Chronic 3 | Hazardous to the aquatic environment, chronic toxicity, category 3 |
| H225 | Highly flammable liquid and vapour. |
| H226 | Flammable liquid and vapour. |
| H341 | Suspected of causing genetic defects. |
| H361d | Suspected of damaging the unborn child. |
| H301 | Toxic if swallowed. |
| H311 | Toxic in contact with skin. |
| H331 | Toxic if inhaled. |
| H370 | Causes damage to organs. |
| H302 | Harmful if swallowed. |
| H304 | May be fatal if swallowed and enters airways. |
| H373 | May cause damage to organs through prolonged or repeated exposure. |
| H314 | Causes severe skin burns and eye damage. |
| H318 | Causes serious eye damage. |
| H319 | Causes serious eye irritation. |
| H315 | Causes skin irritation. |
| H317 | May cause an allergic skin reaction. |
| H336 | May cause drowsiness or dizziness. |
| H371 | May cause damage to organs. |
| H400 | Very toxic to aquatic life. |
| H410 | Very toxic to aquatic life with long lasting effects. |
| H411 | Toxic to aquatic life with long lasting effects. |

# BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO

Printed on 03/12/2024 34/35

| H412 | Harmful to aquatic life with long lasting effects. |
| --- | --- |
| EUH066 | Repeated exposure may cause skin dryness or cracking. |

LEGEND:

- ADR: European Agreement concerning the carriage of Dangerous goods by Road

- ATE: Acute Toxicity Estimate

- CAS: Chemical Abstract Service Number

- CE50: Effective concentration (required to induce a 50% effect)

- CE: Identifier in ESIS (European archive of existing substances)

- CLP: Regulation (EC) 1272/2008

- DNEL: Derived No Effect Level

- EmS: Emergency Schedule

- GHS: Globally Harmonized System of classification and labeling of chemicals

- IATA DGR: International Air Transport Association Dangerous Goods Regulation

- IC50: Immobilization Concentration 50%

- IMDG: International Maritime Code for dangerous goods

- IMO: International Maritime Organization

- INDEX: Identifier in Annex VI of CLP

- LC50: Lethal Concentration 50%

- LD50: Lethal dose 50%

- OEL: Occupational Exposure Level

- PBT: Persistent, bioaccumulative and toxic

- PEC: Predicted environmental Concentration

- PEL: Predicted exposure level

- PMT: Persistent, mobile and toxic

- PNEC: Predicted no effect concentration

- REACH: Regulation (EC) 1907/2006

- RID: Regulation concerning the international transport of dangerous goods by train

- TLV: Threshold Limit Value

- TLV CEILING: Concentration that should not be exceeded during any time of occupational exposure.

- TWA: Time-weighted average exposure limit

- TWA STEL: Short-term exposure limit

- VOC: Volatile organic Compounds

- vPvB: Very persistent and very bioaccumulative

- vPvM: Very persistent and very mobile

- WGK: Water hazard classes (German).

GENERAL BIBLIOGRAPHY

- Regulation (EC) 1907/2006 (REACH) of the European Parliament

- Regulation (EC) 1272/2008 (CLP) of the European Parliament

- Regulation (EU) 2020/878 (II Annex of REACH Regulation)

- Regulation (EC) 790/2009 (I Atp. CLP) of the European Parliament

- Regulation (EU) 286/2011 (II Atp. CLP) of the European Parliament

- Regulation (EU) 618/2012 (III Atp. CLP) of the European Parliament

- Regulation (EU) 487/2013 (IV Atp. CLP) of the European Parliament

- Regulation (EU) 944/2013 (V Atp. CLP) of the European Parliament

- Regulation (EU) 605/2014 (VI Atp. CLP) of the European Parliament

- Regulation (EU) 2015/1221 (VII Atp. CLP) of the European Parliament

- Regulation (EU) 2016/918 (VIII Atp. CLP) of the European Parliament

- Regulation (EU) 2016/1179 (IX Atp. CLP)

- Regulation (EU) 2017/776 (X Atp. CLP)

- Regulation (EU) 2018/669 (XI Atp. CLP)

- Regulation (EU) 2019/521 (XII Atp. CLP)

- Delegated Regulation (UE) 2018/1480 (XIII Atp. CLP)

- Regulation (EU) 2019/1148

- Delegated Regulation (UE) 2020/217 (XIV Atp. CLP)

- Delegated Regulation (UE) 2020/1182 (XV Atp. CLP)

- Delegated Regulation (UE) 2021/643 (XVI Atp. CLP)

- Delegated Regulation (UE) 2021/849 (XVII Atp. CLP)

- Delegated Regulation (UE) 2022/692 (XVIII Atp. CLP)

- Delegated Regulation (UE) 2023/707

- Delegated Regulation (UE) 2023/1434 (XIX Atp. CLP)

- Delegated Regulation (UE) 2023/1435 (XX Atp. CLP)

- Delegated Regulation (UE) 2024/197 (XXI Atp. CLP)

| Suarez Company | Revision nr. 1 Dated 03/12/2024 First compilation Printed on 03/12/2024 35/35 |
| --- | --- |
| BLK0276-2 - SWEET HOME - PROFUMATORE AMBIENTE SANDALO |  |
| - The Merck Index. - 10th Edition - Handling Chemical Safety - INRS - Fiche Toxicologique (toxicological sheet) - Patty - Industrial Hygiene and Toxicology - N.I. Sax - Dangerous properties of Industrial Materials-7, 1989 Edition - IFA GESTIS website - ECHA website - Database of SDS models for chemicals - Ministry of Health and ISS (Istituto Superiore di Sanità) - Italy Note for users: The information contained in the present sheet are based on our own knowledge on the date of the last version. Users must verify the suitability and thoroughness of provided information according to each specific use of the product. This document must not be regarded as a guarantee on any specific product property. The use of this product is not subject to our direct control; therefore, users must, under their own responsibility, comply with the current health and safety laws and regulations. The producer is relieved from any liability arising from improper uses. Provide appointed staff with adequate training on how to use chemical products. CALCULATION METHODS FOR CLASSIFICATION Chemical and physical hazards: Product classification derives from criteria established by the CLP Regulation, Annex I, Part 2. The data for evaluation of chemical-physical properties are reported in section 9. Health hazards: Product classification is based on calculation methods as per Annex I of CLP, Part 3, unless determined otherwise in Section 11. Environmental hazards: Product classification is based on calculation methods as per Annex I of CLP, Part 4, unless determined otherwise in Section 12. |  |